Please add the following notes to the front cover page of the engineering plans:

All construction, materials, and workmanship shall conform to the latest standards and practice of the City of Vancouver and the 2014 Edition of the “Standard Specifications for Road, Bridge, and Municipal Construction” as prepared by WSDOT and APWA.

City of Vancouver Transportation Services Standard Details dated 7/1/14 shall be utilized in the construction of the transportation elements of these plans.

Street signing and striping shall be installed by the Developer.  All street signs and striping shall be installed per the MUTCD.

All construction within City of Vancouver or Clark County right-of-way shall have an approved traffic control plan and right-of-way permit prior to any on-site construction activity.

Street lighting will be installed by the Developer per P.U.D. approved street lighting plans.

Pre-paving as-builts shall be submitted to the City of Vancouver Construction Office and City inspector for both sanitary sewer and storm sewer, prior to paving.

Paving will not be allowed during wet or cold weather, per W.S.D.O.T. specifications.

Any significant deviations from the plans will require a request from the applicant’s engineer and approval from the City’s engineer and City inspector.

All pavement shall be straight cut prior to paving.  Existing pavement shall be removed as necessary to provide a smooth transition for both ride and drainage.

All ADA pedestrian ramps shown on the plans and on the detail sheets shall be constructed with the project.  Where the sidewalk ends at the property line, a pedestrian ramp shall be provided to accommodate wheel chair access.  Sidewalks shall be constructed with the project where there is an existing house, drainage swale, etc. or along an arterial that does not allow access from the new lots.

Subgrade preparation during wet or winter time construction is usually/often not feasible.  A wet or winter time plan shall be submitted to City of Vancouver, Development Engineering staff for review and approval if the contractor plans to commence with construction during wet weather conditions.  If paving from October 15th to March 30th, a wet weather subgrade preparation plan is required.  The subgrade must be over excavated and a geotextile liner used.  The inspector shall approve a complete proof roll test on both sides of the street.

All traffic signal interconnect conduits and cables (copper or fiber optic) shall be protected during construction activities.  Due to the importance of maintaining these communications, any damages to these cables and conduits caused by the contractor or any of its affiliates shall be reported within 2 hours to Operations Counter Dispatch at (360) 696-8177 and repaired within 48 hours unless otherwise approved by City Traffic Engineer.  If this repair can not be completed in allotted time, work will be done by the city or its designee and all costs including any overhead costs shall be invoiced to the contractor.

All traffic signals shall remain in operation during construction activities, except as indicated on the plans.  Any damages caused by the contractor or any of its affiliates to the existing traffic signal conduit, wiring, poles, mast arms, signal indications, loop detectors, and other related components shall be reported within 24 hours unless otherwise approved by engineer.  If this repair can not be completed in allotted time, work will be done by the city or its designee and all costs including any overhead costs shall be invoiced to the contractor.

The developer shall consider measures that provide un-interrupted and full operation of the traffic signal(s) located at all intersections at all times during the construction of the project.  These measures shall include vehicle detections and pedestrian movements on all approaches in a way that normal operation continues; for instance installation of video detection before loop detections are severed.  The developer shall coordinate with City of Vancouver’s inspectors for traffic signal modifications and/or any other activity that would impact the normal operation of the City’s traffic system as part of the temporary traffic control.

All damaged loop detections shall be replaced within 48 hours unless otherwise approved by the City of Vancouver’s inspector or traffic engineer.

Contractor shall report all damages immediately to the City’s Construction Services office at (360) 487-7750 or contact the inspector on the job.

Should any item of archaeological interest (VMC 20.710.090) be found during development, you are required to stop work and notify the planning case manager in Development Review Services at (360) 487-7800, and the Washington State Office of Archaeology and Historic Preservation at (360) 753-4011 immediately.  Failure to do so could result in a felony conviction.

An alternate pedestrian accessible route of travel is required when an existing accessible route is blocked during construction.  The alternate accessible route shall meet minimum accessible standards as set forth in the latest version of the American Disabilities Act Accessibility Guidelines Manual.