Electricity Subcommittee

Summary of Work Plans Recommended for Quantification

| Work Plan | Work Plan Name | Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  | GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs (NPV, | Cost-Effectiveness |
|  |  | (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | Million $) | ($/tCO2e) |
| Electricity 1 | Act 129 of 2008 (HB 2200) (Already in Electricity Baseline Forecast) | 4 | -258 | -65 | 40 | -1409 | -35 |
| Electricity 2 | Reduced Load Growth | 7 | -432 | -64 | 23 | -849 | -36 |
| Electricity 3 | Stabilized Load Growth | 9 | -593 | -64 | 27 | -990 | -36 |
| Electricity 4 | Alternative Energy Portfolio (Act 213 of 2004) Tier I Standard (Already in Electricity Baseline Forecast) | 14 | 680 | 50 | 114 | 3569 | 31 |
| Electricity 5 | House Bill 80: Carbon Capture and Sequestration in 2014 | 5 | 291 | 58 | 13 | 391 | 31 |
| Electricity 6 | Improve Coal-Fired Power Plant Efficiency by 5% | 5 | 82 | 1 | 55 | 903 | 1 |
| Electricity 7 | Sulfur Hexafluoride (SF6) Emission Reductions from the Electric Power Industry | 0.1 | 0.1 | 0.6 | 0.7 | 0.3 | 0.4 |
| Electricity 8 | Analysis to Evaluate Potential Impacts Associated with Joining Regional Greenhouse Gas Initiative | See Text |  |  |  |  |  |
| Electricity 9 | Promote Combined Heat and Power (CHP) | 4 | 53 | 12 | 23 | 209 | 9 |
| Electricity 10 | Nuclear Capacity | 15 | 832 | 57 | 31 | 655 | 21 |
| Electricity 11 | Greenhouse Gas Performance Standard for New Power Plants | Qualitative Workplan--Not Quantified |  |  |  |  |  |
| Electricity 12 | Transmission and Distribution Losses | Qualitative Workplan--Not Quantified |  |  |  |  |  |
|  | Sector Total After Adjusting for Overlaps | 32 | 1080 | $ 33 | 131 | 1862 | $ 14 |
|  | Reductions From Recent Actions | - | - | - | - | - | - |
|  | Sector Total Plus Recent Actions | 32 | 1080 | $ 33 | 131 | 1862 | $ 14 |

GHG = greenhouse gas; MMtCO2e = million metric tons of carbon dioxide equivalent; $/tCO2e = dollars per metric ton of carbon dioxide equivalent; NPV = net present value; TBD = to be determined.

Negative values in the Cost and the Cost-Effectiveness columns represent net cost savings.

The numbering used to denote the above draft work plans is for reference purposes only; it does not reflect prioritization among these important draft work plans.

Electricity 1. Act 129 (HB 2200) Work Plan for Potential GHG Reduction Measure

Strategy Name: Act 129 of 2008 (HB 2200)

Lead Staff Contact: Joe Sherrick (717-772-8944)

Summary:  This initiative identifies the carbon emissions benefits associated with the reduction of electricity consumption and peak load, as described in Act 129 of 2008.  Act 129 requires:

- A reduction in electricity consumption, by May 31, 2011 of 1.0% below consumption levels for the period June 1, 2009 through May 31, 2010.

- A reduction in electricity consumption, by May 31, 2013 of 3.0% below consumption levels for the period June 1, 2009 through May 31, 2010 (additional reduction of 2.0% from the June 2009 through May 2010 baseline for a net total reduction of 3.0%).

- A reduction in peak demand, by May 31, 2013 of 4.5% of the highest 100 hours of demand.  Note: The costs and benefits of this aspect of Act 129 have not been quantified.  Please see the assumptions section below for rationale.

Please note that the imposition of requirements of Act 129 is not inclusive of the very modest consumption and associated system losses from municipalities that are service providers or the rural electric cooperatives.

Other Involved Agencies: PUC has implementation responsibility

Possible New Measure(s):

A report from the American Council for an Energy Efficient Economy (ACEEE) has been drafted for the PUC and DEP and  provides the cost and supply data for the workplan.  Act 129 does not specify how these reductions are to be achieved.  Responses will be purely market-driven.

Workplan Costs and GHG Reductions:

Table 1.1 Workplan Cost and GHG Results

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 4.0 | $ (258) | $ (65) | 39.8 | $ (1,409) | $ (35) |

Notes: The cost estimates (colums 3 and 6) are incremental costs of energy efficient measures including capital cost, operating and maintenance, and labor, above baseline measure costs.  The cost estimates are calculated as the costs less avoided energy expenditures. Also, the difference between the 2020 cost effectiveness (column 4) and the cumulative cost effectiveness (column 7) is due, in part, to the effects of discounting the net cash flows over the analysis period of 2009-2020.

The net present value of the cost savings resulting from implementation of Act 129 from 2009-2020 is estimated at approximately $1.7 billion.  Some of this will be due to peak load reductions that result in lower wholesale energy and capacity charges but not less energy used.  (These are not quantified in this draft). Peak demand reductions are assumed to not have an impact on GHG emissions as noted below.  There is the assumption that lower wholesale charges will be passed through to customers.  Other savings will result through reducing energy consumption.

Quantification Approach and Assumptions

- Reductions from the workplan are assumed to begin in 2009-2011 and implemented at 0.33% per year through 2011 to achieve the 1% target by 2011.  Reductions are then assumed to be 1% per year for 2012 and 2013 reaching the Act 129 target of 3%.

- GHG mitigation and costs from the peak demand reduction component of Act 129 are not quantified as recommended by the subcommittee.

  - The costs and GHG reduction compliance pathways are deemed to uncertain for us to be able to quantify.  For instance, peak demand reductions could be met with peak shifting from peak periods where the marginal resource is natural gas turbines, to off peak periods where the baseload resource is coal which has a higher CO2 emissions intensity (tons/MWh).  Other peak reductions might arise from the energy efficiency deployment obtained under the other components of Act 129.  The costs of compliance equipment such as smart meters and associated communications equipment that might also be used to meet the peak demand reduction are also deemed to uncertain to quantify.

- Statewide load forecast from the PUC are used as the basis for the calculations.  This includes the load reduction effects of Act 129 (which are already in the baseline), so reductions estimated here are likely to be slightly understated (by 3% of 3%).

- The above efficiency percentage targets are applied to residential, commercial and industrial loads.  The cost and supply of efficiency savings are thus dependent on the customer class load as a percent of total load.  Industrial loads grow more slowly than residential and commercial through 2020, and thus over time a smaller share of efficiency savings comes from the industrial sector.

- Energy efficiency costs are expressed as levelized costs over the life of the energy efficiency options over the planning period. The incremental costs (typically incurred in the first year of program implementation) are spread over all future years of the life of the energy efficiency measures.

- Efficiency investments installed under Act 129 with expected lifetimes shorter than the planning period are expected to be replaced with equipment with similar cost and performance characteristics.  Efficient equipment is cost effective to install initially, and it is assumed that it will be replaced at the end of its life.  Thus the electricity reductions in 2013 under Act 129 are held steady through 2030.

- The cost of the workplan is calculated by estimating the annual costs of energy efficiency less avoided electricity expenditures. These cash flows are then discounted at a real rate of 5%.

  - The net present value of cash flows is calculated beginning in 2009 through 2020.

- All prices are in $2007 as per the Center for Climate Strategies Quantification Memo.

|  | 2009 |  |  |
| --- | --- | --- | --- |
| Levelized Cost of Energy Efficiency ($2007) | $/MWh | $/MMBTU | Fixed Cost Rate |
| Residential | $ 53.70 | $ 5.68 | 13% |
| Commercial | $ 31.47 | $ 3.52 | 10% |
| Industrial | $ 26.03 | $ 2.11 | 5% |

  - Sum of Capital and Fixed Costs Program fixed costs are assumed to be part of each measure’s capital cost, These include administrative, marketing, and evaluation costs of 5%.

- Source: ACEEE et al. (2009).Various pages

- Cost of EE measures includes program and participant cost as is typically used in Total Resource Cost test.

| Costs Associated with Avoided Energy in 2009 ($2007) | $/MWh | $/MMBTU |
| --- | --- | --- |
| Residential | 103.37 | 13.14 |
| Commercial | 87.14 | 10.72 |
| Industrial | 65.00 | 7.48 |

- For electricity, retail end user prices for Jan 09 from US EIA Monthly Electricity Profile, increased by 6.2% in 2010 to account for rate caps coming off for last of EDCs. Annual prices in 2011+ adjusted by change in AEO end user prices from table 74 of AEO 2009 supplemental tables. http://www.eia.doe.gov/cneaf/electricity/epm/table5_6_a.html

- For natural gas, retail annual 2008 prices by sector, annual changes from 2009 onwards from Table 12 of AEO 2009 regional tables http://tonto.eia.doe.gov/dnav/ng/ng_sum_lsum_dcu_SPA_m.htm and http://www.eia.doe.gov/oiaf/aeo/supplement/stimulus/regionalarra.html

- The costs to implement Act 129 are recoverable by utilities, so customers will be funding the efficiency deployment.

- Based on the costs of energy efficiency per MWh above, annual spending in 2013 will be approximately $177 million.

- Electricity transmission and distribution losses are assumed to be 6.6% over the analysis period.  Source: PA Electricity Inventory and Forecast.xls

- To estimate emission reductions from workplans that are expected to displace conventional grid-supplied electricity (i.e., energy efficiency and conservation) a simple, straightforward approach is used.  We assume that these policy recommendations would displace generation from an “average thermal” mix of fuel-based electricity sources of  coal and gas.  This mix is based on 90% coal, 10%  gas for all years 2009-2030 based on EIA 2006 State Electricity Profile data.

  - The average thermal approach is preferred over alternatives because  sources without significant fuel costs would not be displaced—e.g., hydro, nuclear, or renewables generation.

    - Similarly, a “marginal” approach is not possible in Pennsylvania because the natural gas share of the annual generation portfolio (13.5 million MWh) of total generation (218 million MWh in 2006) is only about 6%.  This small amount does not provide enough be “backed down” due to the energy efficiency deployment in the workplan.

  - Given the generation fleet’s coal and gas combustion efficiencies, this equates to a CO2 intensity of approximately 0.87 tonnes/MWh. This compares to the average  statewide CO2 intensity of 0.54 tonnes/MWh (including hydro, nuclear, etc.)

  - This approach provides a transparent way to estimate emission reductions and to avoid double counting (by ensuring that the same MWh from a fossil fuel source are not “avoided” more than once). The approach can be considered a “first-order” approach; it does not attempt to capture a number of factors, such as the distinction between peak, intermediate, and baseload generation; issues in system dispatch and control; impacts of nondispatchable and intermittent sources, such as wind and solar; or the dynamics of regional electricity markets. These relationships are complex and could mean that policy recommendations affect generation and emissions (as well as costs) in a manner somewhat different from that estimated here. Nonetheless, this approach provides reasonable first-order approximations of emission impacts and offers the advantages of simplicity and transparency that are important for stakeholder processes.

    - Note that some renewable resources, like cofiring biomass with coal or dedicated biomass gasification do  have substantial fuel costs.  However, because these resources are negligible in the reference case electricity supply forecast, they are not able to be “backed down” in the analysis.

- Cost to DEP - None

- Cost to the Commonwealth – Administrative

- Cost to regulated community or consumer – Act 129 requires only modest reductions in load growth.  It is reasonably anticipated that consumers will realize long-term cost savings however, there are costs of implementation that will be bore by the rate base and will be quantified in filings to the PUC.  Estimated gross cost savings are provided at the end of this work plan and will need to be reconciled with the implementation costs.

- Are there Federal funds available? – N/A

- Do these costs fund other programs? N/A

- Are cost savings realized from this initiative? Yes, as noted above.  Market forces will drive compliance options and the path forward.  Actual savings will likely vary widely among the EDC territories, within the various rate classes and economic sectors and also based on socio-economic factors for residential consumers.

Implementation Steps:

- Act 129 was signed into law on October 15.

- By January 15, 2009, the PUC must adopt an energy efficiency and conservation program that requires each electric distribution company (EDC) to develop and implement cost effective energy efficiency & conservation plans to reduce consumption and peak load within their service territories, as noted in the Summary section of this work plan.

- The American Council for an Energy Efficient Economy (ACEEE) has conducted a statewide assessment of cost effective energy efficiency potential.    For potential follow up workplans to build on Act 129, see workplans Electricity 2 and 3.

Potential Overlap:

- See Appendix B for Overlap Analysis

Electricity 2. Reduced Load Growth Work Plan for Potential GHG Reduction Measure

Strategy Name: Reduced Load Growth

Lead Staff Contact: Joe Sherrick (717-772-8944)

Summary:  This initiative identifies the carbon emissions benefits associated with curbing the rate of growth in electricity consumption in PA.  This strategy builds upon the conservation requirements of Act 129 of 2008, which requires 1.0% and 2.0% reductions in electricity consumption from 2010, by 2011 and 2013, respectively.  Act 129 also requires the PUC to assess the potential for additional cost-effective reductions.  The scenario developed in this work plan builds upon Act 129 by requiring biennial reductions in electricity consumption equal to 1.5% per  period (.75% per year), beginning in 2015 and carrying through 2025.  The energy efficiency investments under this workplan therefore reach 8.25% of load by the end of 2025 (11 years at 0.75% per year). These reductions are calculated from the previous years estimated consumption.

Please note that this analysis does not include the very modest consumption and associated system losses from municipalities that are service providers or the rural electric cooperatives.

Other Involved Agencies: PUC

Possible New Measure(s): A report from the American Council for an Energy Efficient Economy (ACEEE) has been drafted for the PUC and DEP and provides the cost and supply data for the workplan.  See: http://www.aceee.org/pubs/e093.htm

Workplan Costs and GHG Reductions:

Table 2.1 Workplan Costs and GHG Results ($2007)

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 6.7 | $ (432) | $ (64) | 23.3 | $ (849) | $ (36) |

The net present value of the cost savings resulting from implementation of this workplan from 2009-2020 is estimated at approximately $930 million.  The cost savings and emissions reductions are additional to Act 129.  The cost savings are more modest compared to Act 129 because the workplan is not implemented until 2015 and has reached efficiency investments equal to 4.5% of sales by 2020. These distant cash flows are then discounted back to the present time.

Notes: The cost estimates (colums 3 and 6) are incremental costs of energy efficient measures including capital cost, operating and maintenance, and labor, above baseline measure costs.  The cost estimates are calculated as the costs less avoided energy expenditures. Also, the difference between the 2020 cost effectiveness (column 4) and the cumulative cost effectiveness (column 7) is due, in part, to the effects of discounting the net cash flows over the analysis period of 2009-2020.

- Cost to DEP - None

- Cost to the Commonwealth – Act 129 requires the PUC to hire a program administrator to oversee this process and to provide assessments as to the cost-effectiveness and level of additional reductions that may be possible within PA.  The cost for this service is unknown.

- Cost to regulated community or consumer  To the extent that this workplan mirrors the funding mechanisms of Act 129, utility costs, up to a portion of revenues will be recoverable, so customers will be funding the entire cost of the workplan up to that level. The ACEEE et al (2009) report assumes that a portion of the cost of each efficiency measure may be spent by the end user and that utility incentives comprise the balance of the initial costs, but that these incentives will be funded by customers.

- Based on the costs of energy efficiency per MWh (discussed in Electricity 1), annual spending in 2020 will be approximately $300 million.

- Are there Federal funds available? – Federal funding is not required nor is it available at this time.  Limited assistance may be available through the U.S. DOE State Energy Plan but this would most likely be limited to policy analysis and possibly technical support.

- Do these costs fund other programs? No.  Any costs are expected to result in changes to consumer behavior.

Quantification Approach and Assumptions

- Reductions from the workplan are assumed to begin in 2015 and implemented at 0.75% per year through 2025 to achieve  a rate of 8.25% by 2025.

- Efficiency investments installed under the workplan with expected lifetimes shorter than the planning period are expected to be replaced with equipment with similar cost and performance characteristics.  Efficient equipment is cost effective to install initially, and it is assumed that it will be replaced at the end of its life.  Thus, the electricity reductions in 2025 under the workplan are held steady through 2030.

- For cost and other assumptions see Electricity #1—Act 129.

Implementation Steps:  The following, and other, considerations that could be examined as policy tools to support this measure:

- Act 129 provides the PUC with the necessary authority to require additional cost-effective reductions in electricity consumption.

- An assessment of electricity consumption reduction potential is necessary to determine if the requirements suggested within this work plan conform to Act 129 requisites.

- A legislative amendment to the AEPS establishing a dedicated market share for energy efficiency credits (new tier or carve out) might facilitate achieving this reduction measure by rewarding over-compliance and providing a cost-effective manner to achieve greater reductions.

- Require electric distribution companies to invest in demand side response initiatives, including rebates to consumers.

- Require that all cost-effective supply side and demand side response initiatives have been identified and acted upon before approvals for new generation are granted.  In a February 5, 2007 press release http://www.aceee.org/press/e072pr.htm The ACEEE reported that, “States from Texas to Vermont are finding energy efficiency resources available at less than 4 cents per kilowatt-hour, compared to the expected cost of power from new plants of 5 to 10 cents.”

- Implementing advanced building standards for the commercial, institutional, state and municipal government sectors that establish minimum green building practices and energy efficiency standards.

- Coupled with the advanced building standard, consider a requirement that all commercial, institutional, state and municipal government buildings perform annual benchmarking, similar to that established by the U.S. EPA that documents the gains or losses in energy consumption on a per square foot basis, based on the type of activity occurring.  This information is publicly accessible.  It educates and encourages building owners and operators to achieve higher performance.

- Work with neighboring states on establishing regional efficiency standards for appliances and electronics where none currently exist or where minimum standards are less than optimal.

- Establish an aggressive phase-out of incandescent lights and/or establish a pricing/tax structure that preferentially treats lighting with a higher lumens to watts ratio.

- Rate decoupling and incentives from residential and commercial workplanRC-12.

- Consumer barriers to implementing energy efficiency

Potential Overlap:

- See Appendix B for overlaps.

Electricity 3. Stabilized Load Growth Work Plan for Potential GHG Reduction Measure

Strategy Name: Stabilized Load Growth

Lead Staff Contact: Joe Sherrick (717-772-8944)

Summary:  This measure builds upon the very modest reductions required via Act 129 of 2008.  Act 129 requires reductions in consumption of 1.0% by 2011 and 2.0% by 2013, for a total of 3%, measured against 2010 consumption.  The Stabilized Load Growth (SLG) scenario further investigates the potential impact of annual consumption reductions of 0.75% per year in the period 2015 through the end of 2017 followed by a rate of consumption that is held static from 2018 through 2025.  Historical annual load growth in PA has been approximately 1.5% per year which is what would be reduced in the 2018 to 2025 period.  .  The energy efficiency investments under this workplan therefore reach 14.4% of load by the end of 2025 (2015-2017 at 0.75% per year, 2018 at .85% per year, 2019-2025 at 1.6% per year). The annual reductions in 2018-2025 would be based on the previous year’s consumption figures and would allow a subsequent one year “true-up” for electricity distribution companies to achieve stabilized consumption levels.

Please note that this analysis does not include the very modest consumption and associated system losses from municipalities that are service providers or the rural electric cooperatives.

The demand reductions under this workplan can be compared to those occurring in other jurisdictions.  Several states are mandating energy savings akin to the higher performers in Figure 3.1.  Iowa’s PUC has requested utilities file plans to achieve savings equal to 1.4% of sales, up from 0.8 currently.  New York has a target of 15% savings by 2015, which was started in 2007 equating to new energy efficiency investments equal to nearly 2% per year. The following figure shows incremental energy savings as a percent of sales for surveyed utilities across the country.

Figure 3.1: Energy Savings as % of First Year Sales

Other Involved Agencies: PUC

Possible New Measure(s): A report from the American Council for an Energy Efficient Economy (ACEEE) has been drafted for the PUC and DEP and provides the cost and supply data for the workplan.  See: http://www.aceee.org/pubs/e093.htm

Workplan Costs and GHG Reductions:

Table 3.1 Workplan Costs and GHG Results ($2007)

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 9.2 | $ (593) | $ (64) | 27.2 | $ (990) | $ (36) |

The net present value of the cost savings resulting from implementation of this workplan from 2009-2020 is estimated at approximately $ 1.4 billion.  The cost savings and emissions reductions are additional to Act 129.

Notes: The cost estimates (colums 3 and 6) are incremental costs of energy efficient measures including capital cost, operating and maintenance, and labor, above baseline measure costs.  The cost estimates are calculated as the costs less avoided energy expenditures. Also, the difference between the 2020 cost effectiveness (column 4) and the cumulative cost effectiveness (column 7) is due, in part, to the effects of discounting the net cash flows over the analysis period of 2009-2020.

- Cost to DEP - None

- Cost to the Commonwealth – Act 129 requires the PUC to hire a program administrator to oversee this process and to provide assessments as to the cost-effectiveness and level of additional reductions that may be possible within PA.  The cost for this service is unknown.  It is further assumed that the PUC would perform similar services to oversee the reductions that may be required if such a Stabilized Load Growth initiative were to be implemented.

- Cost to regulated community or consumer  To the extent that this workplan mirrors the funding mechanisms of Act 129, utility costs, up to a portion of revenues will be recoverable, so customers will be funding the entire cost of the workplan up to that level. The ACEEE et al (2009) report assumes that a portion of the cost of each efficiency measure may be spent by the end user and that utility incentives comprise the balance of the initial costs, but that these incentives will be funded by customers.

- Based on the costs of energy efficiency per MWh (discussed in Electricity 1), annual spending in 2020 will be approximately $415 million.

- Are there Federal funds available? – Federal funding is not required nor is it available at this time.  Limited assistance may be available through the U.S. DOE State Energy Plan but this would most likely be limited to policy analysis and possibly technical support.

- Do these costs fund other programs? No.  Any costs are expected to result in changes to consumer behavior.

- Are cost savings realized from this initiative? Cost savings are expected but this does require a detailed analysis.  The assumption is that reductions will only be required such that can be sustained through cost-effective measures.

Quantification Approach and Assumptions

- Reductions from the workplan are additional to those under Act 129, and are assumed to begin in at the start of 2014 and are implemented  through the end of 2017 at 0.75% of sales per year (for a total of 3% of sales). This reduction is expected to reduce Pennsylvania’s load growth rate from ~1.60%/year to about 0.85%/year. Then required reductions are equal to the load growth rate from the previous year from 2018 through 2025.  By 2020, expected reductions are equal to approximately 6.3% of sales, and by 2025 reductions amount to 14.4% of sales.

- Efficiency investments installed under the workplan with expected lifetimes shorter than the planning period are expected to be replaced with equipment with similar cost and performance characteristics.  Efficient equipment is cost effective to install initially, and it is assumed that it will be replaced at the end of its life.  Thus, the electricity reductions in 2025 under the workplan are held steady through 2030.

- For cost and other assumptions see Electricity #1—Act 129.

Additional Assumptions:

- Adequate cost-effective reductions exist or will exist, through year 2025, to provide the approximate 27 million MWh of curtailment, as compared to the unchecked, projected rate of growth in electricity consumption. The ACEEE report identifies cost effective efficiency supplies in Table 3.2 of approximately 61 million MWh that significantly exceed the reductions projected under this workplan.

Table 3.2: Summary of Cost-Effective Energy Efficiency Potential by Sector (2025)

- No reductions would be required if not supported through an analysis of cost-effective measures.

Implementation Steps:  The following, and other, considerations should be examined as policy tools to support this measure:

- Act 129 provides the PUC with the necessary authority to require additional cost-effective reductions in electricity consumption.

- The PUC may have the authority within Act 129 of 2008 that would facilitate realization of this reduction measure.

- A legislative amendment to the AEPS establishing a dedicated market share for energy efficiency credits (new tier or carve out) might facilitate achieving this reduction measure by rewarding over-compliance and providing a cost-effective manner to achieve greater reductions.

- Require electric distribution companies to invest in demand side response initiatives, including rebates to consumers.

- Require that all cost-effective supply side and demand side measures have been identified and acted upon before approvals for new generation are granted.  In a February 5, 2007 press release http://www.aceee.org/press/e072pr.htm ACEEE reported that, “States from Texas to Vermont are finding energy efficiency resources available at less than 4 cents per kilowatt-hour, compared to the expected cost of power from new plants of 5 to 10 cents.”

- Implementing advanced building standards for the commercial, institutional, state and municipal government sectors that establish minimum green building practices and energy efficiency standards.

- Coupled with the advanced building standard, consider a requirement that all commercial, institutional, state and municipal government buildings perform annual benchmarking, similar to that established by the U.S. EPA that documents the gains or losses in energy consumption on a per square foot basis, based on the type of activity occurring.  This information is publicly accessible.  It educates and encourages building owners and operators to achieve higher performance.

- Work with neighboring states on establishing regional efficiency standards for appliances and electronics where none currently exist or where minimum standards are less than optimal.

- Establish an aggressive phase-out of incandescent lights and/or establish a pricing/tax structure that preferentially treats lighting with a higher lumens to watts ratio.

- Rate decoupling and incentives from residential and commercial RC-12 workplan.

- Consumer barriers to implementing energy efficiency

Potential Overlap:

- See Appendix B for list of overlaps between workplans

Electricity 4. Alternative Energy Portfolio (Act 213 of 2004) Tier I Standard: Work Plan for Potential GHG Reduction Measure

Lead Staff Contact: Joe Sherrick (717-772-8944)

Summary:  Identifies GHG reductions associated with the existing AEPS Tier I requirement at 8%.

Other Involved Agencies:  PUC and DEP have shared roles in administering the AEPS

Existing Measure: The AEPS requires that all electricity consumed within PA by 2021 be comprised of at least 0.5% solar PV, 8% from other renewable (Tier I) sources and 10% from other alternative energy (Tier II) sources.  The AEPS matures in 2021 after which, no further increase in renewable generation is required but the standards from 2021 remain in effect.

Projected GHG Reduction:

There could be some CO2 additional reductions through Tier II from sources such as large hydro and energy efficiency.  The contribution of these resources to meeting the Tier II obligation is somewhat uncertain because we already know that sufficient credits from waste coal have been generated to meet the entire Tier II requirements through at least 2021.   The impact is that little incentive exists for the generation of electricity from new, zero carbon emitting sources due to the over-supply created by waste coal.  For the 2007/2008 compliance period, the weighted average Tier II compliance credit traded for $.66.  This amount is too small to affect plant investment decisions. Because of the minimal value of credits associated with Tier II, it is assumed that the waste coal generation that is used to meet compliance with the AEPS would have happened without the regulation.

Hydroelectric – Uprates or upgrades to hydroelectric power generation can come from the addition of incremental (new) generation at existing plants or simply by making improvements in efficiency to things such as turbine design or improvements to the electrical generators.  With the enactment of the Alternative Energy Portfolio Standard (AEPS), such improvements are being given serious consideration by generating companies.  Therefore, it is important to note that if these improvements are made or incremental generation is brought online, the resultant emissions reduction that might accrue will be accounted for under Tier I of the AEPS, provided that these hydroelectric plants obtain certification from the Low Impact Hydro Institute (LIHI), as required under the AEPS.  Any improvements or incremental generation from a hydroelectric plant that does not or can not obtain LIHI certification will earn Tier II credits under the AEPS but the emissions reductions would not count against our total reductions from the AEPS.  Upgrading older hydropower generating systems is common practice in North America.  Through rehabilitation, hydroelectric producers are increasing capacity and efficiency at existing facilities that are several decades old.  Rewinding a generator or replacing a turbine runner can result in performance that not only equals, but also surpasses the capabilities of the equipment when it was new. Rehabilitating existing plants is often a more economical way of adding capacity, when compared to building new facilities.

Workplan Costs and GHG Reductions:

Table 4.1 Workplan Cost and GHG Results

| Annual Results (2020) |  |  | Cumulative Results (2007-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 13.7 | $ 680 | $ 50 | 114.0 | $ 3,569 | $ 31 |

Notes: The cost estimates (colums 3 and 6) are incremental costs of energy efficient measures including capital cost, operating and maintenance, and labor, above baseline measure costs.  The cost estimates are calculated as the costs less avoided energy expenditures. Also, the difference between the 2020 cost effectiveness (column 4) and the cumulative cost effectiveness (column 7) is due, in part, to the effects of discounting the net cash flows over the analysis period of 2009-2020.

Quantification Approach and Assumptions

The costs and GHG reductions from the AEPS are the difference between what is assumed to occur between the AEPS-case and the No AEPS-case.   In the No-AEPS case, the new resources that would have been deployed are assumed to be 90% existing pulverized coal, 10% natural gas peaking gas.  In the AEPS-case, the resources assumed to be deployed are listed in Table 5.2

- Only the cost and GHG benefits from Tier I resources are quantified under this workplan.

  - For the 2007/2008 compliance period, the weighted average Tier II compliance credit traded for $.66.  This amount is too small to affect plant investment decisions. Because of the minimal value of credits associated with Tier II, it is assumed that the waste coal generation that is used to meet compliance with the AEPS would have happened without the regulation.

TABLE 4.2: Tier One Resources Assumed to be Deployed in 2020 under the AEPS

| Tier 1 alternative energy gross generation assumptions (% of New Renewable Resources) | 2010 | 2020 |
| --- | --- | --- |
| Other Gases (CMM) | 0% | 0% |
| Petroleum | 0% | 0% |
| Nuclear | 0% | 0% |
| Hydroelectric (micro, large) | 30% | 15% |
| Geothermal | 0% | 0% |
| Solar/PV | 4% | 4% |
| Wind | 39% | 68% |
| MSW | 19% | 9% |
| Landfill Gas | 5% | 2% |
| Biomass | 2% | 1% |
| Other wastes | 0% | 0% |

- The generation resources that are assumed to be avoided under this workplan are 90% existing pulverized coal, and 10% existing peaking gas.  The weighted average cost of generation for the avoided mix is $49.15 in 2020. The avoided CO2 emissions associated with this mix is .86 tonnes CO2 /MWh.

- While the other technologies are large, central station generation source, the Tier I photovoltaic carve-out is distributed generation.  As such, it has a different avoided cost assumption because PV also avoids new transmission, distribution, and capacity. THe PV carve-out assumes an avoided cost based on the weighted average retail price of electricity for residential, commercial, and industrial customers.  PV generation in 2020 to meet the .5% target in the AEPS is assumed to be 758 GWh, with an avoided cost of $96.67.

- See Appendix C for generation cost assumptions and sources.

- All hydro that is deployed under the AEPS is assumed to be small hydro.  This is a conservative assumption as small hydro costs more than large hydro.

- Cost to DEP – Administration of programs for the continued support of energy efficiency and renewables, particularly solar PV (ex. Energy Harvest, PEDA, Alternative Energy Investment Act, etc.)

- Cost to the Commonwealth – Continued support of renewables, particularly solar

- Cost to regulated community or consumer – Distribution companies pass compliance costs on to the rate-payers.  Until all of the EDC rate caps are removed the impact will remain uncertain.  The removal of the rate caps will have a far more pronounced impact on electricity rates than will the requirements of the AEPS.

- Are there Federal funds available? Stimulus funds from the ARRA (2008) are potentially available for renewables development, as well as federal tax credits production tax credits and investment tax credits.  USDA Farm Bill appropriations can and have provided limited support.  Moreover, as the total appropriations are increasing, the amount available via grant funding is being significantly scaled back in favor of loans.

- Do these costs fund other programs? No.

- Are cost savings realized from this initiative? Not directly.  Indirect savings to the Commonwealth will accrue subject to in-state low carbon electricity development (manufacturing, installation, sales and service, etc.).  Indirect costs include displaced coal industry jobs and other fossil fuel related economic production and consumption.

- Costs quantified in these workplans consider only microeconomic costs and benefits.  The macroeconomic costs and benefits including employment impacts and

Implementation Steps:

- Already being implemented.

- Legislation continues to be drafted that would require additional increases in the amount of alternative energy.  Pennsylvania has the lowest percentage requirements of any surrounding state renewable portfolio standards.  Because the geographic scope from which projects may be considered eligible (Illinois to North Carolina) for Act 213 compliance is much broader than was originally intended and in order to ensure that more renewable energy and associated new jobs are created in PA, the requirements of the AEPS could be increased.

- Act 1 incentives for renewable resources

- Federal production tax credit

Potential Overlap:

- See Appendix B for Overlap Analysis

Electricity 5. Work Plan for House Bill 80

Note: Replaces Tier 1 at 15%, Tier 1 at 20%, Tier 3: Carbon Capture and Sequestration Work plans

Lead Staff Contact: Joe Sherrick

Summary:  Carbon capture retrofit to existing supercritical pulverized coal plants per the requirements in HB80 (as referred to the Committee on Environmental Resources and Energy on March 12, 2009) starting in 2015 through 2019.  In 2020, installation of Integrated Coal Gasification Combined Cycle (IGCC) plant in the state.  We assume a IGCC with capture schedule of 600 MW beginning in 2020 based on typical IGCC plant capacity proposals in states such as Minnesota (Excelsior Energy), Washington (Energy Northwest), and the Ohio Valley (AEP).

Other Involved Agencies: Pennsylvania Public Utility Commission.

Possible New Measure(s):

Retrofits of existing supercritical pulverized coal plants entail amine scrubbing  with a CO2 capture rate of 90% and an increase in heat rate requirements of 31.3%.  The reduction in efficiency is compensated by an increase in capacity of existing plant as the amine scrubbing system  diverts steam for power generation or consumes additional power for CO2 compression.

Integrated Coal Gasification Combined Cycle (IGCC) power plants use coal fuel an input to produce electricity. The technology is based around a gasifier that produces a mixture of hydrogen and carbon monoxide called syngas. This syngas is burned in a gas turbine which is used to drive a generator. Much like in NGCC power plants the turbine exhaust is used in a heat recovery generator to create steam to drive a steam turbine generator. Below is a block diagram of a 500MWe IGCC plant with CO2 capture (MIT, 2007).

IGCC technologies with CO2 capture are equipped with three more processes than the conventional IGCC technology without capture. The first is a process of reacting syngas with steam to produce CO2 and hydrogen through shift reactors. The second process separates the CO2 from the remaining gas. The final process compresses and dries the CO2.

Adding CO2 capture technology to IGCC plants has a significant impact on overall plant efficiency as evidenced in the figure to the right (MIT, 2007).

Workplan Costs and GHG Reductions:

Avoided emissions are calculated on the basis of known potential uprates and new plant build displacing a mix of 90% coal and 10% gas at a combined average of 1,872 lb/MWh.  We assume a base case in which 90% of CO2 emissions are sequestered, though there is substantial uncertainty regarding the long-term leakage of CO2 in various sequestration configurations.  Higher leakage would reduce the cost effectiveness of carbon capture for reducing greenhouse gas emissions.

Table 5.1: Workplan Costs and GHG Results ($2007)

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 5.0 | $ 291 | $ 58 | 12.6 | $ 391 | $ 31 |

- The avoided generation costs are for a 90% coal/10% gas mix.

Table 5.2: Carbon Capture Technology Assumptions for Year 2020

| IGCC with Carbon Capture Characteristics | $2007 | Source |
| --- | --- | --- |
|  | New Plant |  |
| Unit Size MW | 600 MW | Based on numerous IGCC proposals including Excelsior (Minnesota), AEP (Ohio Valley), and Energy Northwest (Washington). |
| Heat Rate MBTU/MWh | 10,334 | Congressional Research Service, p. 97. |
| Capacity Factor | 85% | Congressional Research Service, p. 97 |
| Installed Capital Costs $/kW | $ 4,662.61 | Congressional Research Service, p. 97 |
| O&M Costs $/MWh | $11.51 | Congressional Research Service, p. 97 |
| Economic Life/years | 50 | Assumption |
| Fuel $/MBTU | $ 2.02 | U.S. EIA, AEO 2009 (April 2009 update related to federal stimulus), Table 12 |
| Net Generation Cost $/MWh | $ 98.12 | Calculation |
| Avoided Price of Power $/MWh | $49.15 | Calculation based on existing 90% new coal and 10% gas plant mix. |
| MW Capacity | 600 |  |
| MWh Generation | 4,467,600 |  |

The above technology assumptions include the cost of both the IGCC plant as well as carbon capture equipment and operations.  The Congressional Research Service study bases IGCC minus carbon capture costs on a survey of five IGCC plant proposals throughout the U.S., including the Edwardsport plant in Indiana and the Mountaineer plant in West Virginia.  Carbon capture equipment costs are based on applying a 43% adder, which in turn is based on U.S. EIA estimates of carbon capture capital costs above those for stand-alone IGCC plants.  O&M costs are based on CRS’s review of U.S. EIA’s 2008 long-term forecast.

Given the site-specific nature of sequestration configurations, and given the lack of sufficient operational experience in carbon capture worldwide, the above cost figures may not reflect the actual cost of carbon capture in sites in Pennsylvania.

Table 5.3: Carbon Capture Retrofit Technology Assumptions for Year 2020

| IGCC with Carbon Capture Characteristics | $2007 | Source |
| --- | --- | --- |
|  | New Plant |  |
| Unit Size MW | 267 | Based on HB80 load-serving requirements |
| Heat Rate MBTU/MWh | 15,817 | Congressional Research Service, p. 97. |
| Capacity Factor | 85% | Congressional Research Service, p. 97 |
| Installed Capital Costs $/kW | $2,141 | Congressional Research Service, p. 97 |
| O&M Costs $/MWh | $13.12 | Congressional Research Service, p. 97 |
| Economic Life/years | 50 | Assumption |
| Fuel $/MBTU | $ 2.02 | U.S. EIA, AEO 2009 (April 2009 update related to federal stimulus), Table 12 |
| Net Generation Cost $/MWh | $85.52 | Calculation |
| Avoided Price of Power $/MWh | $ 49.15 | Calculation based on existing 90% new coal and 10% gas plant mix. |
| MW Capacity | 267 | Based on HB80 load-serving requirements |
| MWh Generation | 1,987,492 |  |

The above costs and heat rate are based on the Congressional Research Service’s review of the 2007 MIT study The Future of Coal.  O&M costs are based on a review by CRS of the National Energy Technology Laboratory’s study of the Conesville plant in Ohio.

The assumed capacity of retrofits to existing supercritical pulverized coal plants is based on requirements in HB80 as referred to the Committee on Environmental Resources and Energy on March 12, 2009.  The bill requires that regulated load-serving entities source a maximum of 3% of total electric energy sold to retail customers in the state from coal-fired plants with carbon capture, as a part of the Tier II tranche of resources.  The bill’s language does not ramp up the maximum from carbon capture for subsequent years, even through the overall Tier II requirement does go up over time. Thus, the energy requirement would grow only based on load growth.

The bill states that acceptable "coal combustion with limited carbon emissions" is a plant that captures 40% of its CO2 from 2015-2019, 60% from 2019-2024, and 90% from 2024 onwards. We apply those percentages to overall existing coal generation in the state.  

The bill contains numerous provisions for load-serving entities, including triggering force majeure if carbon capture does not materialize in the wholesale electricity market.  The bill also includes allowance of long-term contracts with carbon capture plants provided numerous cost-effectiveness tests are met, including overall price of energy, price of capacity, and price of alternative energy credits.

Future Fuels has proposed a 150-MW IGCC plant near Good Spring, Pa. (Schuykill County) to be supplied by anthracite from a nearby mine.

- The above analysis assumes a 90% capture (10% leakage) rate consistent with the Congressional Research Service report.  However, the electricity subcommittee was also interested in a sensitivity analysis of the costs with higher leakage rates.

  - Assuming a 50% capture rate, the 2020 cost per ton of CO2e mitigated rises to $104/ton with a 2020 reduction of 2.8 MMTCO2e.  Cumulative costs (2009-2020) are estimated at 7 MMTCO2e, with a discounted cost of $56/ton.

Economic Cost: Market forces will drive Investments into infrastructure, to uprate capacity.  These upfront costs will yield greater energy generation capacity and efficiency, leading to increased sales and eventually, increased profits.

Implementation Steps:  The following, and other, considerations could be examined as policy tools to support this measure:

- Leveraging Federal stimulus funds for CCS, which amounts to $3.5 billion and when combined with existing Federal funds (primarily from the Energy Policy Act of 2005) results in $8 billion in total Federal support for CCS.

- CCS portfolio requirements for load-serving entities, similar to what the state of Illinois has supported, which is set at 5% with a cap on overall rate impacts.

- Loan guarantees for early-stage development of CCS infrastructure to reduce financing costs closer to government borrowing rates.

- Funding for technicial assessments of CCS potential in the state.

- Investment tax credits to cover up-front capital costs.

- Production tax credits over a specified period of generation.

- Direct cost sharing of project development costs through appropriations.

- Streamlined permitting for generation and associated transmission.

- This analysis mirrors the language in HB 80 relating to the 2015 start date for deploying carbon capture and sequestration.  Given the long lead times for this and other developing technologies, there is considerable uncertainty regarding the timing, technical issues, permitting, and financing associated with retrofitting existing pulverized coal plants with carbon capture and sequestration.

Potential Overlap:

- See Appendix B for Overlap Analysis

Electricity 6. Improve Coal-Fired Power Plant Efficiency by 5% Work Plan for Potential GHG Reduction Measure

Lead Staff Contact: Krish Ramamurthy (717-772-3369)

Summary: Require a 5% increase in energy efficiency at coal-fired power plants by 2025.  Each facility would have the flexibility to meet this efficiency requirement at the least cost method available to them.  Measure is assumed to be implemented linearly in 2015 following scheduled outage in PJM que.

Other Involved Agencies:  Workplan measures would need to be designed so as not to trigger “Major Modification” clause in US EPA New Source Review (NSR) program for major stationary sources in attainment areas.   NSR requires plant owners to undergo review for environmental controls in case of major modifications beyond routine maintenance, repair and replacements.  Determination of what measures trigger NSR is made on a case-by-case basis, with numerous efforts by U.S. EPA to create more broad guidelines to inform plant owners what measures trigger NSR.  One provision that is currently delayed by U.S. EPA until at least 2010 is how numerous physical and operational changes are aggregated in determining whether the measures trigger NSR.  The delayed rule, originally issued on January 15, 2009, determined that such changes can be aggregated only if they are “substantially related” and occur within three years of the other changes.  However, the recent delay points to continued case-by-case determination of if and how numerous changes trigger NSR.  This analysis includes design and operational changes that may or may not trigger NSR.  The analysis avoids modeling added plant capacity associated with efficiency improvements as one effort to avoid assumptions that would more likely trigger NSR.

The typical methods that could be utilized for compliance with this measure are listed in the table from the Australian Greenhouse Gas Office publication below.  This analysis excludes the table’s list of “retrofit improvement” measures as an attempt to screen measures that are more likely to be considered to be “major modifications” under NSR.

Possible New Measure(s): An affected Electricity Generating Unit (EGU) may improve efficiency to minimize system losses as a means to reduce CO2 emissions.  For instance, a 15 percent increase in efficiency at an EGU would result in a 13% decrease in CO2 emissions.  Upgrades can include improvements to the boiler, turbine, and control systems.  Examples of turbine improvements include installing high efficiency turbine blades, which allow for increased power generation and an efficiency improvement of 0.98%.  Fuel consumption reduction can occur with improvements to feed water heater material within a turbine system, leading to an increase in efficiency of between 1% and 5%.  Upgrading the software of the control system that monitors and fine-tunes combustion can improve efficiency between 0.3% to 3%.

Table 6. Workplan Cost and GHG Results

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 5.4 | $82.1 | $1.5 | 55.4 | $903.4 | $1.5 |

Quantification Approach and Assumptions

- The measures selected in the analysis draw upon the Australian Greenhouse Gas Office study detailed below, with a cross-reference check with the National Energy Technology Laboratory’s Reducing CO2 Emissions By Improving the Efficiency of the Existing Coal-Fired Power Plant Fleet (July 2008), which also lists potential efficiency improvement measures, though without associated cost information.  The measures, listed in order of lowest cost on a CO2 reduction basis to highest cost, are:

  - Reducing turbine gland leakage (0.84% efficiency improvement)

  - Refurbishing feedheaters (1% efficiency improvement)

  - Improving combustion control (0.84% efficiency improvement)

  - Reducing steam leaks (1.1% efficiency improvement)

  - Lowering excess air operation (1.22% efficiency improvement)

- Whether all the above measures can be implemented in a single plant is dependent upon plant-specific physical and operational conditions.  Further, whether all measures can be implemented with additive efficiency benefits is also a plant-specific determination.  The analysis avoided including multiple measures affecting a single aspect of a plant (e.g., numerous feedheater-related measures) to avoid overlapping measures as best as possible.

- The result of the measures is to improve heat rate efficiency, thereby reducing CO2 emissions from existing plant capacity.  While the Australian study lists the total efficiency of the above measures at 4.84%, we draw upon NETL’s study which lists ranges of efficiency improvements from the above measures, and increase the efficiency benefit of feedheater refurbishment from 0.84% (as listed in the Australian study) to 1%, which is within the range of potential efficiency improvement cited in the NETL study, to reach a total of 5% efficiency improvement.

- Costs are based on the Australian study’s estimate of cost per unit of reduced CO2 emissions.  The Australian study assumes an 8% discount rate over 25 years.

- Implementation is assumed to affect all existing coal-fired generation in the state beginning in 2010.

- Cost to DEP – The cost to DEP will be in terms of staff man hours invested in developing any new regulation, or guidance document, that will be required for this effort.  Also, if additional conditions need to be added to permits this will require additional staff time invested by regional office personnel.

- Cost to regulated community or consumer – A study conducted by the Australian Greenhouse Office (January 2000) evaluated the costs and benefits of efficiency improvements to electric generating units.  This paper can be found at http://www.environment.gov.au/settlements/ges/publications/pubs/skmreport.pdf

- The availability of federal funds for such improvement projects is unknown.

- The cost to other programs at the federal level is not known.

- The cost of the measures that fall under this workplan are potentially understated should the modifications trigger NSR, which would then require additional pollution control measures at the retrofitted plants.

- Another potential source of information on efficiency improvements at existing coal plants is the McKinsey report from Dec 2007 …How much at what cost?

The table below, from the Australian Greenhouse Office (January 2000) Report, Integrating Consultancy Efficiency Standards for Power Generation, illustrates the cost in terms of tons of CO2 reduced for a variety of power plant efficiency improvement steps.  For each efficiency improvement action the cost can be determined based on the expected reduction of carbon dioxide in terms of tons of CO2 reduced.  All data in this table is in terms of Australian dollars and metric tones.

Potential Overlap:

- See Appendix B for Overlap Analysis

Electricity 7. Sulfur Hexafluoride (SF6) Emission Reductions from the Electric Power Industry Work Plan for Potential GHG Reduction Measure

Lead Staff Contact: Krish Ramamurthy (717-783-9476)

Summary:  This initiative uses a pollution prevention approach, including a best management practices manual and recordkeeping and reporting requirements, to ensure that all SF6 emission reductions are quantified and permanent.

Other Involved Agencies: United States Environmental Protection Agency

Possible New Measure(s): SF6 is identified as the most potent non-CO2 greenhouse gas with the ability to trap heat in the atmosphere 23,900 times more effectively than CO2. Approximately 80% of SF6 gas produced is used by the electric power industry in high voltage electrical equipment as an insulator or arc quenching medium. SF6 is emitted to the atmosphere during various stages of the equipment’s life cycle. Leaks increase as equipment ages. The gas can also be accidentally released at the time of equipment installation and during servicing.

Emissions estimates for Pennsylvania include:

| Basis | Year | SF6 Emissions | (MMTCO2e) |  |
| --- | --- | --- | --- | --- |
| CIRA-2003 | 1990 | SF6 from Electric Utilities | 0.8 | 87% |
| CIRA-2003 | 1990 | SF6 from Magnesium | 0.1 | 13% |
|  |  | Total | 0.9 | 100% |
| CIRA-2003 | 1999 | SF6 from Electric Utilities | 0.9 | 76% |
| CIRA-2003 | 1999 | SF6 from Magnesium | 0.3 | 24% |
|  |  | Total | 1.2 | 100% |
| PEC-2007 | 1990 | SF6 from Electric Utilities | 1.2 |  |
| PEC-2007 | 2000 | SF6 from Electric Utilities | 0.6 |  |
| PEC-2007 | 2020 | SF6 from Electric Utilities | 0.3 |  |

A regulatory program could be developed in Pennsylvania that uses pollution prevention approach including a best management practices manual and recordkeeping and reporting requirements to ensure that all SF6 emission reductions are quantified and permanent. The reduction of SF6 emissions from the electric power industry is available as one of the offset opportunities for any Cap and Trade Program established for large emitters under RGGI.

As part of this regulatory program, a manual could be developed that would identify best management practices that would be required of all owners and operators of electric power systems. Best management practices could include proper handling techniques, identification and elimination of leaks, and the replacement of equipment that do not meet specific leak rate thresholds. An example of best management practices would be the recent Duquesne Light Company decommissioning of an old substation to recover the SF6 gas and reclaim it to ASTM standards. The project resulted in the removal of approximately 7,300 lbs of SF6 that otherwise would have been emitted to the atmosphere. As a part of SF6 Emission Reduction Partnership for Electric Power Systems, Exelon’s PECO subsidiaries set a SF6 goal in March 2006, commit a leak rate for SF6 of no more than 10% for 2006. To help achieve the companies provided additional training to substation personnel to minimize SF6 gas leaks and revised the gas handling procedures.   Annual recordkeeping and reporting requirements would be required to ensure the quantification and reduction of SF6 emissions.

Workplan Costs and GHG Reductions:

The EPA identifies several categories of reduction measures.  The following text is from the EPA website:

- Recycling Equipment

  - The capital costs of recycling equipment range from around $5,000 to over $100,000 per utility. For this analysis, typical recycling expenditures have been set at $25,500 per utility. However, this capital  investment produces O & M savings of nearly $1,600 per year per utility due to reduced purchases of SF6.

- Leak Detection and Repair

  - There are no capital costs associated with leak detection and repair and O&M costs are estimated to be $2,190 per utility due to the increased labor costs associated with this option.

- Equipment Replacement/Accelerated Capital Turnover

  - The capital costs of this option vary by equipment type. Circuit breakers (below 34.5 kV) may be replaced with vacuum breakers. The replacement cost varies from $25,000 to $75,000 per unit. Medium and high voltage breakers are expected to continue to use SF6 because no other option is currently available. Older breakers are assumed to leak more and are being replaced by new equipment (as part of routine turnover) at a cost of approximately $200,000 to $750,000 per unit. Additional research into the existing equipment stock and potential for replacement will be necessary to develop cost estimates for emission reductions.

- Advanced Leak Detection Technologies

  - The capital cost per GasVue leak detection camera is approximately $100,000. Additional research into the potential emission reductions from this option will be necessary to develop estimates for O&M costs and the total cost of emission reductions.

Summary of Measures and Costs

The most promising options to reduce SF6 emissions from electric power systems are SF6 recycling and

SF6 leak detection and repair. SF6 recycling could reduce emissions by about 10 percent and is currently

cost-effective. Leak detection and repair could reduce emissions cost-effectively by 20 percent.

Actual EPA partnership experience shows that even greater reductions have been experienced. The 2007

annual report shows that partner emissions rates have decline by nearly 60 percent from over 15 % of consumption  to 5.5%.

Table 7.1 Workplan Cost and GHG Results

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 0.1 | $ 0.1 | 0.59 | 0.73 | 0.29 | 0.39 |

Quantification Approach and Assumptions

- The SF6 program is assumed to be implemented linearly over a 5 year period beginning in 2012.  By the end of 2016, SF6 reductions are assumed to be 30% of forecasted emissions from the electricity sector.  The reductions are split into 20% leak detection and 10% recycling.

  - Note that future reductions could be much larger than this based on actual experiences by SF6 partner utilities between 2000-2007.

- The cost estimates employ an 8% discount rate, a ten-year project lifetime, and an SF6 price of $8.00 per pound.  Mitigation costs for leak detection are estimated at $.44/ton CO2e and recycling equipment at $.90/ton CO2e.

- SF6 emissions from the electric power sector are estimated at  .6 MMTCO2e in 2000 and at .3 MMTCO2e in 2020.   Emissions in the interim period are linearly interpolated.  Emissions are held constant at 2020 levels through 2030.

Industry:  Mitigation of emissions are cheaper than purchasing new SF6 supplies.  These benefits are not quantified here for lack of specific cost data.

Department: No cost authorized or anticipated.  Development of any regulatory program would then be required to be accomplished through existing resources and budget.

Funding Sources: US EPA voluntary cooperative program is implemented under Federal funding independent of Pennsylvania’s budget process.

Implementation Steps: US EPA voluntary cooperative program is implemented and summarized at http://www.epa.gov/electricpower-sf6/. Pennsylvania’s major power producers are participants.

Potential Overlap:

- N/A

Electricity 8. Join Regional Greenhouse Gas Initiative Work Plan for Potential GHG Reduction Measure

Lead Staff Contact:  PUC and DEP

Initiative Summary: Examine the potential CO2 emissions reductions associated with joining the Regional Greenhouse Gas Initiative.

Other Involved Agencies: PUC and DEP

Possible New Measure(s):

RGGI is composed of individual CO2 Budget Trading Programs in each participating state. These programs are implemented through state regulations, based on a RGGI Model Rule, and are linked through CO2 allowance reciprocity. Regulated power plants are able to use a CO2 allowance issued by any of the participating states to demonstrate compliance with the state program governing their facility. Taken together, the individual state programs function as a single regional compliance market for carbon emissions. The RGGI Model Rule may be found at http://www.rggi.org/docs/Model%20Rule%20Revised%2012.31.08.pdf

To reduce emissions of greenhouse gases, the RGGI participating states are using a market-based cap-and-trade approach that includes:

- Establishing a multi-state CO2 emissions budget (cap) that will decrease gradually until it is 10% lower than at the start

- Requiring electric power generator to hold allowances covering their emissions of CO2

- Providing a market-based emissions auction and trading system where electric power generators can buy, sell and trade CO2 emissions allowances

- Using the proceeds of allowance auctions to support low-carbon-intensity solutions, including energy efficiency and clean renewable energy, such as solar and wind power

- Employing offsets (greenhouse gas emissions reduction or sequestration projects at sources beyond the electricity sector) to help companies meet their compliance obligations

RGGI's phased approach means that reductions in the CO2 cap provide predictable market signals and regulatory certainty. Electricity generators will be able to plan for and invest in lower-carbon alternatives and avoid dramatic electricity price impacts.

The RGGI target is to hold state CO2 emissions from the power sector constant at 2009 levels until 2014.  Beginning in 2015, a 2.5% reduction per year for four years through the end of 2018, at which time capped emissions are targeted at 10% below 2009 emissions.

The following table shows the forecasted Pennsylvania business-as-usual (BAU) emissions, corresponding RGGI target, and in the final row shows the required reductions to meet the target.  Note that the effects of energy efficiency investments required under Act 129 (2008) are included in the BAU emissions forecast, as are the renewable energy requirements from Pennsylvania’s Alternative Energy Portfolio Standard (2004).

Table 8.1 Pennsylvania Forecasted Emissions and the RGGI Targets 2009-2020

| Electricity Sector Emissions--Million Metric Tons CO2 Equivalent (MMTCO2e) | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total (Production-Based) | 115 | 117 | 120 | 122 | 123 | 125 | 126 | 128 | 129 | 131 | 133 | 134 |
| Total (Consumption-Based—Not used in analysis | 83 | 85 | 86 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 97 |
| RGGI CAP | 115 | 115 | 115 | 115 | 115 | 115 | 112 | 109 | 107 | 104 | 104 | 104 |
| Required Reductions From BAU (Production Based Emissions less RGGI Cap) | - | 2.3 | 4.5 | 6.4 | 8.0 | 9.6 | 14.0 | 18.4 | 22.9 | 27.4 | 29.0 | 30.5 |

Although the first RGGI compliance target ends in 2018, this analysis considers emissions and reductions out to 2020 because this is the Pennsylvania Climate Change Advisory Council’s terminal analysis year.

Pennsylvania’s BAU emissions are forecasted to grow by over 1.5 million tonnes per year between 2005 and 2020.  This equates to an increase in emission of 10 MMTCO2e between 2009 and 2014, after which the 2.5% annual reductions are required.  Between 2015 and 2020, Pennsylvania’s power sector emissions are forecasted to grow an additional 9 MMTCO2e.

By 2020, the forecast predicts that RGGI compliance would require approximately 30 MMTCO2e reductions from the electricity sector.  There are two categories of reductions that need to occur to meet the RGGI target:

- Reduce the 2009-2020 forecasted BAU emissions increase of 19 MMTCO2e to hold state emissions constant at 2005 levels.

- Reduce the additional 11 MMTCO2e required to reach the 10% below 2005 RGGI target.

Several conclusions can be drawn from this analysis regarding potential RGGI compliance. First, Pennsylvania’s emissions growth needs to begin to slow immediately in order to for the state to realistically meet the RGGI target.  Because of their low cost and short lead times, demand side management measures are the optimal choice to stabilize emissions levels. Second, in the longer term, reductions in the carbon intensity of the electricity generated in Pennsylvania will be required to meet the RGGI targets. Finally, these two considerations should be viewed as a portfolio of reductions in the electricity sectors.  Cost savings (negative cost measures) from demand side options can be viewed to “pay” for higher cost fuel switching measures on the supply side.  While DSM requires capital outlays that are typically paid for by consumers, these investments costs less than new supply side investments, and mitigate cost increases from low carbon generation, as well as transmission and distribution investments.

Potential Costs and Supplies of GHG Emissions Reductions for Pennsylvania

Modeling of the costs to the state from joining RGGI proceeds in a stepwise fashion. The approach is to aggregate the statewide GHG emissions reductions that are grid connected. First, is an analysis of the reductions in GHG emissions from reduced electricity consumption.

Table 8.2: Demand Side GHG Reductions Identified in CCAC Workplans

|  |  | Annual Results | (2020) |
| --- | --- | --- | --- |
| Work Plan No. | Work Plan Name | GHG Reductions (MMtCO2e) | Cost Effectiveness ($/tCO2e) |
| Electricity 3 | Stabilized Load Growth (Industrial Sector Only) | 3 | $ (64.43) |
| RC-1 | High Performance State and Local Government Buildings | 2 | TBA |
| RC-2 | High Performance School Buildings | 1 | TBA |
| RC-3 | High Performance Commercial (private) Buildings | 5 | TBA |
| RC-4 | High Performance Homes (Residential) | 11 | TBA |
| RC-5 | Commission Buildings | 1 | TBA |
| RC-6 | Re-Light PA | 6 | TBA |
| RC-8 | Appliance Standards | 1 | TBA |
| RC-9 | Geothermal Heating and Cooling | TBA | TBA |
|  | Total Demand Side Reductions | 30 | TBA |

- The costs of RGGI compliance are likely to be dominated by the negative cost energy efficiency (demand side) measures.  A study conducted by the University of Maryland (January 2007) evaluated the costs and benefits of participating in the Regional Greenhouse Gas Initiative.  This study can be found at http://www.cier.umd.edu/RGGI/documents/UMD_RGGI_STUDY_Jan07.pdf

The main conclusions of this study indicate that, overall, joining RGGI would only have a limited impact on the economy and electric power markets in Maryland.  Similar conclusions hold for the current RGGI region and affected areas outside this region.

Electricity Bill Impacts in MD:  Overall, electricity bills are forecast to decrease over $100 million in 2010 and more than $200 million by 2025.  This is a result of energy efficiencies, which will lower customers’ demands.  Since the heaviest users will save the most, more than half the savings (between 53% and 63%) will go to industrial and commercial customers.  On average, a residential ratepayer will see a modest reduction – about $22 savings in 2010 per household.

Overall Economic Impacts in MD:  Will have little net impact on the Maryland economy.  The positive economic impacts from reduced electricity costs and energy efficiency investments are partially offset by reduced investment and profits in the electricity generating sector.  Overall RGGI is predicted to have a positive economic impact on Gross State Product of approximately $100 million in 2010, increasing to about $200 million in 2015 and subsequent years.  This impact is expected to create approximately 1200 jobs across the state by 2010, increasing to 2800 jobs by 2025.  Such positive impacts are less than 0.1% of overall Maryland gross state product and employment in all years.

  - The costs to Pennsylvania are not necessarily reflective of the  above modeled costs to Maryland.

Table 8.3: Low Cost Supply Side GHG Reductions Identified in CCAC Workplans

|  |  | Annual Results (2020) |  |
| --- | --- | --- | --- |
| Work Plan No. | Work Plan Name | GHG Reductions (MMtCO2e) | Cost Effectiveness ($/tCO2e) |
| Waste 1 | Landfill Methane Displacement of Fossil Fuels | 0.1 | $ (0.80) |
| Waste 5 | Waste-to-Energy Digesters | 0.1 | $ 1.00 |
| Waste 6 | Waste-to-Energy MSW | 0.24 | $ (34.00) |
| Forestry 8 | Wood to Electricity | 0.26 | $ 0.67 |
| Forestry 9 | Combined heat and power | 0.47 | –$45.30 |
| Ag-4 | Ag Digesters (Methane) | 0.20 | $ (0.25) |
| Electricity 6 | Improve Coal-Fired Power Plant Efficiency by 5% | 5 | $ 15.21 |
| Electricity 7 | Sulfur Hexafluoride (SF6) Emission Reductions from the Electric Power Industry | 0.1 | $ 0.59 |
| Electricity 9 | Promote Combined Heat and Power (CHP) | 4 | $ 12.20 |
| Electricity 10 | Nuclear (Uprates Only) | 4 | $ 19.72 |
|  | Total Supply Side Reductions | 15 | $ 13.44 |

These electricity supply options do not include new renewables supplies and fuel switching from coal-to-gas.  For instance, HB 80 sets new targets for the AEPS following 2021.  This was not quantified because the HB80 requirements begin after the CCAC 2020 planning horizon.

Offsets

Another source of supplies for RGGI compliance comes from offsets. The RGGI program has included flexibility mechanisms to limit costs to the regulated sector.  One of these mechanisms creates offset allowances from CO2 mitigation projects outside of the power sector.  Offsets are initially allowed in the program up to 3.3% of an entity’s compliance obligation.  If annual average allowance prices exceed $7 (in $2005), then this amount increases to 5%, and if annual allowance prices exceed $10, then this amount increases to 10%. At the 10% level, international CO2 reduction credits are also allowed. In the reference case 3.3% of obligations, total offsets allowed by Pennsylvania entities in 2020 would be approximately 4.4 MMTCO2e.

The following list identifies the categories of offset projects currently allowed, and representative costs/to of CO2.  The costs are approximate and are taken from the relevant CCAC subcommittee workplans dated 6/15/2009 or later:

- Landfill methane capture and destruction (-$1/ton)

- Reduction in emissions of sulfur hexafluoride (SF6) in the electric power sector ($2/ton)

- Sequestration of carbon due to afforestation (-$10/ton)

- Reduction or avoidance of CO2 emissions from natural gas, oil, or propane end-use combustion due to end-use energy efficiency in the building sector (-$25/ton)

- Avoided methane emissions from agricultural manure management operations (-$1/ton)

The offset accreditation process will likely entail some administrative costs that are not included in the above CCAC costs. Given the low or negative costs of the above measures, plus accreditation costs, a generic cost estimate for RGGI offsets is estimated at $5/ton CO2e.  Assuming that the costs of offsets credited in the RGGI program reflect the microeconomic quantification for the CCAC process, then they could exhibit a significant downward cost of compliance for regulated actors.

Summary

The above categories of costs and supplies are summarized in Table 8.4.

| Category of Measures | PA Supply of GHG Reductions (MMtCO2e) | Cost Effectiveness ($/tCO2e) | Comment |
| --- | --- | --- | --- |
| Demand Side | 30 | $ (10.00) | Placeholders pending overlaps and cost information |
| Supply Side (CCAC) | 15 | $ 13.44 | Includes overlaps |
| Offsets | 4.4 | $ 5.00 | 3.3% cap on offsets. See text for cost information |
| Total | 50 | $ (1.58) | Weighted average cost/ton |

Limitations and Uncertainties

- As of 2007, Pennsylvania is a large exporter of electricity.  The reference case GHG forecast assumes this will continue through 2020 as the growth in electricity generation is equivalent to growth in electricity sales.  However, if Pennsylvania cannot site new fossil based generation resources at this rate, then GHG emissions from the power sector will be low than reported here.

- Similarly, the compliance costs estimated above require the timely implementation of policies to develop the GHG reduction measures identified under the CCAC process.

- Other costs: Cost to DEP & PUC – The cost will be in terms of staff man hours invested in developing any new regulation, or guidance document, that will be required for this effort.  Also, additional staff time invested by regional office personnel necessary to update permits.

Quantification Approach and Assumptions

- Emissions reductions required to meet RGGI targets are based on PA production-based inventory which includes all electricity generated, including exported electricity.

- Power sector emissions are assumed to be held constant at 2009 levels through the end of 2014.  Beginning in 2015, emissions are reduced by 2.5% of 2009 levels.

- The generation resources that are assumed to be avoided under this workplan are 90% existing pulverized coal, and 10% existing peaking gas.  The weighted average cost of generation for the avoided mix is $9.15 in 2020. The avoided CO2 emissions associated with this mix is .86 tonnes CO2 /MWh

Implementation Steps: New legislation and new regulation based on RGGI Model Rule is required.

Potential Overlap: See Appendix B for Overlap Analysis

Electricity 9. Promote Combined Heat and Power Work Plan for Potential GHG Reduction Measure

Strategy Name: Promote combined heat and power (CHP)

Lead Staff Contact: Maureen Guttman (717-783-8411)

Summary:  This initiative encourages distributed  CHP systems to reduce fossil fuel use and GHG emissions.  Reductions are achieved through the improved efficiency of CHP systems, relative to separate heat and power technologies, and by avoiding transmission and distribution losses associated with moving power from central generation stations to distant locations where electricity is used.

Other Involved Agencies:

Possible New Measure(s):

Combined heat and power (CHP) is a term used to describe scenarios in which waste heat from energy production is recovered for productive use. The theory of CHP is to maximize the energy use from fuel consumed and to avoid additional GHG emissions from entities a baseload generating station via additional fossil fuel combustion, so that a reliable source of thermal energy can be provided to the users of the reclaimed thermal energy. The reclaimed thermal energy, while can be used by other nearby entities (e.g., within an industrial park or district steam loop) for productive purposes. Generating stations in urban areas may have existing opportunities or may require the co-location of new industry.  For Pennsylvania,  the largest source of new, cost-effective CHP potential is in industrial facilities that have continuous thermal loads for domestic hot water and process heating (ACEEE, et al, 2009).  CHP units are typically sized to the minimum thermal load for the facility.

- Promote use of natural gas-fired CHP.

- Promote use of biomass-fired CHP.

- Create or expand markets for CHP units by incentives designed to promote implementation for residential, commercial and industrial users.

- Promote CHP technologies through provisions for tax benefits, attractive financing, utility rebates and other incentives.

- Remove barriers to CHP development, such as utility rate structures that allow discounted electric rates to compete with CHP.  Also, interconnection standards should be designed to facilitate economical and efficient CHP connection to the grid.

- Consider the economic and environmental benefits of CHP as a resource in each electric utility’s Integrated Resource Plan.  Potential measures include training and certification of installers and contractors; net metering and other pricing arrangements; clear and consistent interconnection standards; and creation and support for biomass fuel markets.

Potential Workplan Costs and GHG Reductions:

Table 9.1 Workplan Costs and GHG Results ($2007)

| Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 4.4 | $ 53 | $ 12 | 23.2 | $ 209 | $ 9 |

The composition of the costs presented in Table 9.1 differ according to the type of CHP.  Commercial CHP has the highest costs, in part because of the relatively low capacity factor (47% in 2010 rising to 64% in 2020) implied in the ACEEE et al (2009) report. These low capacity factors are somewhat unusual because CHP units, especially commercial applications, are typically sized to the meet the constant thermal demand of the facility.  These units are then run at maximum capacity to generate the required thermal output.

The cost and emissions estimates assume three types of technologies are representative of the CHP portfolio in the future. Table 9.2 reflects the assumptions for each technology.

- Biofuel CHP supply: Ethanol and biodiesel production requires the distillation of separating mixtures based on differences in their volatilities in a boiling liquid mixture and thus require significant thermal inputs. The goal of the federal renewable fuels standard of 10.21% for 2009 (11.1 billion gallons of renewable fuel), is required by the Energy Independence and Security Act of 2007 (EISA). The EISA (2007) targets 40 billion gallons by 2022.

- Act 78, a state law passed in July 2008, requires that every gallon of gasoline and diesel fuel contain a percentage of ethanol and biodiesel, respectively. The law targets 20% biodiesel and all gasoline sold at retail must contain10 percent ethanol, once in-state cellulosic ethanol production reaches 350 million gallons.

- The Agricultural subcommittee workplan #2 on advanced biofuels targets 545 million gallons of biofuels being produced in PA by 2020.  This is the target used for the biofuels CHP component of this workplan. This analysis assumes biofuels processing CHP supply is derived from the assumption that the processing of the fuels requires heat inputs equal to 38% of fuel energy content (an energy balance of 2.62, similar to the energy balance of cellulosic ethanol).

  - The biofuels component of the workplan is relatively modest, as exhibited in Table 9.2, installed capacity in 2020 is only estimated at approximately 180 MW.

- The CHP supply estimates in the ACEEE et al (2009) report targets the year 2025.  For interim years such as 2020, supplies are linearly interpolated.  The growth rate for 2026-2030 is 8.3%, 6.0%, and 0% for commercial, industrial, and biofuels processing respectively.

- The avoided CO2 emissions rates are assumed to be the same as in workplan #1..

- The fuel for commercial and industrial and biomass processing CHP is 100% natural gas.

- T&D losses are 6.6%.

- Industrial retail electricity prices are the avoided electric prices for industrial and biofuels CHP.  Commerical retail electricity prices are the avoided electric price for commercial CHP. The avoided CO2 emissions associated with this mix is .86 tonnes CO2 /MWh from a 90% coal, 10% gas mix.

- Estimating the costs of CHP into the distant future is tentative, because cost estimates are highly sensitive to natural gas prices, the cost of avoided power, and the assumption about the CO2 intensity of displaced electricity.

CHP potentials come from ACEEE et al (2009) Table E-14. Market Penetration Results for $500/kW Incentive Case.  This is the aggressive policy case where clean public energy funds subsidize the capital costs to install CHP at a rate of $500/kW.  This quantification incorporates the total social costs, including private and public costs, into the cost per MMCO2e measure.

Table 9.2: CHP Technology Assumptions

Implementation Steps:

The key to implementing CHP systems is to provide adequate incentives for the development of infrastructure to capture and utilize the waste heat. Such incentives could come in many forms, such as recruiting suitable end users to a centralized location to utilize the waste heat, a feed-in tariff for CHP electricity, including CHP electricity in energy efficiency or renewables targets, tax credits, grants, zoning, and offset credits for avoided emissions.

Potential Overlap:

- See Appendix B for Overlap Analysis

Electricity 10. Nuclear  Work Plan for Potential GHG Reduction Measure

Lead Staff Contact: Dan Griffiths (717-773-0542)

Summary:  Capacity uprates at existing nuclear plants in PA. DEP estimates 1,050 MW of additional potential capacity at PA nuclear power plants (Limerick, Peach Bottom, Susquehanna, Three Mile Island).  Of this total, approximately 150 MW is expected to be available by 2012. Of the remaining 900 MW we assume that a bit less than half of the remaining MW capacity will be developed (eg ~400 MW) for a total of 550 MW by 2020.  Therefore, the nuclear uprate schedule for the state is assumed to be 150 MW in 2012, and an addition of 100 MW of capacity in 2014, 2016, 2018 and 2020..  For new plant build, PPL is proposing a 1600 MW Bell Bend plant at the site of the Susquehanna 1 and 2 that is also analyzed under this workplan.

Other Involved Agencies: N/A

Possible New Measure(s):

Nuclear Uprates - To increase the power output of a reactor, typically a more highly enriched uranium fuel is added. This enables the reactor to produce more thermal energy and therefore more steam, driving a turbine generator to produce electricity. In order to accomplish this, components such as pipes, valves, pumps, heat exchangers, electrical transformers and generators, must be able to accommodate the conditions that would exist at the higher power level. For example, a higher power level usually involves higher steam and water flow through the systems used in converting the thermal power into electric power. These systems must be capable of accommodating the higher flows.

In some instances, facilities will modify and/or replace components in order to accommodate a higher power level. Depending on the desired increase in power level and original equipment design, this can involve major and costly modifications to the plant such as the replacement of main turbines. All of these factors must be analyzed by the facility as part of a request for a power uprate, which is accomplished by amending the plant's operating license. The analyses must demonstrate that the proposed new configuration remains safe and that measures continue to be in place to protect the health and safety of the public. Before a request for a power uprate is approved, the Nuclear Regulatory Commission must review these analyses.

Potential GHG Reduction:

Avoided emissions are calculated on the basis of known potential uprates and new plant build displacing a mix of 90% coal and 10% gas at a combined average of 1,872 lb/MWh.

Table 10.1.  Workplan Costs and GHG Results

| ($2007)Annual Results (2020) |  |  | Cumulative Results (2009-2020) |  |  |
| --- | --- | --- | --- | --- | --- |
| GHG Reductions | Costs | Cost-Effectiveness | GHG Reductions | Costs | Cost-Effectiveness |
| (MMtCO2e) | (Million $) | ($/tCO2e) | (MMtCO2e) | (NPV, Million $) | ($/tCO2e) |
| 14.7 | $ 832 | $ 57 | 31.0 | $ 655 | $ 21 |

- Nuclear uprate costs are based on FPL Energy’s proposed uprate of its Florida-based Turkey Point and St. Lucie pressurized water reactor units to be completed in 2011.

Pressurized water reactors exist at the Beaver Valley and Three Mile Island plants in Pennsylvania.

- The generation resources that are assumed to be avoided under this workplan are 90% existing pulverized coal, and 10% existing peaking gas.  The weighted average cost of generation for the avoided mix is $49.15 in 2020. The avoided CO2 emissions associated with this mix is .86 tonnes CO2 /MWh

Table 10.2: Nuclear Technology Assumptions

| Nuclear Characteristics | $2007 | For Year 2020 | Source |
| --- | --- | --- | --- |
|  | New Plant | Uprate |  |
| Unit Size MW | 1,600 | varies | New Plant: PPL’s proposed Bell Bend plant. Uprate: staff assumption based on common unit uprate proposals—e.g., FPL’s proposed 378-uprate proposal for 4 units. |
| Heat Rate MBTU/MWh | 10,400 | 10,400 | ACEEE, et al (2009) p. 212 |
| Capacity Factor | 90% | 90% | Assumption |
| Installed Capital Costs $/kW | $ 7,310.31 | $ 3,892 | New Plant: Climate Strategies ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change. Uprate: FPL proposed 2011 uprate for Turkey Point and St. Lucie plants. |
| O&M Costs $/kWh | $13.33 | $ 3.1 | New Plant: Climate Strategies ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change. Uprate: Same as above, minus fixed O&M costs. |
| Economic Life/years | 50 | 50 | Assumption |
| Fuel $/MBTU | $ 1 | $1 | Climate Strategies ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change |
| Net Generation Cost $/MWh | $ 122.99 | $ 66.20 | Calculation |
| Avoided Price of Power $/MWh |  |  | Calculation based on 90% new coal and 10% new gas plant mix. |
| MW Capacity | 1,600 | 550 | Described Above |
| MWh Generation | 12,614,000 | 3,153,600 | Calculation |

Implementation Steps:

- Market forces will drive Investments into infrastructure, to uprate capacity.  These upfront costs will yield greater energy generation capacity and efficiency, leading to increased sales and eventually, increased profits.

- These actions are currently being implemented

- Market-driven initiative Are cost savings realized from this initiative? Not directly.  Indirect savings to the commonwealth will accrue subject to in-state low carbon electricity development (manufacturing, installation, sales and service, etc.).  Indirect costs include displaced coal industry jobs and other fossil fuel related economic production and consumption.

Potential Overlap:

- See Appendix B for Overlap Analysis

- RGGI Workplan

Appendix A: Incentives Workplan From RC Subcommittee

TBA

Appendix B: Overlap Analysis

| Workplan: | Overlaps With Workplan: | Overlap Adjustment To: | Notes | Resolution |
| --- | --- | --- | --- | --- |
| Electricity -3 Stabilized Load Growth | Electricity -2 Reduced Load growth. | Electricity -2, | Electricity 2 and 3 are substitutes for each other. | Reductions from Electricity 2 are eliminated. |
| Electricity -3 Stabilized Load Growth | Industry-2 Industrial Gas and Electricity | Electricity -3 | Industry 2 targets 9% industrial efficiency by 2020 while Electricity-3 is only 7%. The issue for the interaction between these workplans is not overlaps, but assurance that in combination they do not exceed industrial electric efficiency supplies in PA. By 2020, the combined GWh of both workplans exceeds by approximately 350 GWh the linear implementation of the two 2025 industrial estimates in ACEEE et al (2009) of 9,900 and 13,000 GWh (pp. 14, 30). | 2020 reductions of electric industrial energy efficiency are reduced by 350 GWh (10% of industrial electric efficiency reductions under Electricity 3). |
| Electricity-8 RGGI | Electricity 3, Electricity-9 CHP, Electricity-6 Nuclear, Industry 2-Industrial gas and Electricity | Electricity-8 RGGI | RGGI analysis utilizes a statewide cost curve using other electricity workplans and the estimated renewables supplies in the state or region | This workplan uses the cost curves developed for the CCAC process as well as estimates of new sources of reductions outside the existing workplans (ie new renewables). Biomass requirements for ag, forestry and waste are removed from the supplies assumed available for the RGGI analysis. |
| Electricity -3 Stabilized Load Growth | RC-12 Utility Incentives for Electricity Demand-Side Management | None | It is unclear that decoupling and rate incentives will add incremental reductions to existing targets under Electricity-3. Rather incentives and decoupling are potentially a necessary implementation mechanism for stabilizing load growth. | None required |
| Electricity -3 Stabilized Load Growth | RC-3, RC-4: High Performance Commercial and High Performance Homes (Residential) (private) Buildings | Electricity-3 | 2020 commercial efficiency reductions under RC-3 are estimated at 9,001 GWh versus only 3,300 for Electricity 3. 2020 residential reductions under RC-4 are estimated at 17,541 GWh versus only 3,400 for Electricity 3. | 100% of residential and commercial reductions from Electricity 3 are eliminated due to overlaps |
| Electricity -3 Stabilized Load Growth | RC-12 Commissioning and Retrocommissioning | Electricity-3 | Retrocommissioning accounts for 300 of 17,260 GWh commercial reductions in 2025 in ACEEE et al (2009) p. 143 | All residential and commercial reductions in Electricity 3 were eliminated due to overlaps from RC-3 and RC-4. Commissioning reductions kept in RC-12. |
| Industry-2 Industrial Gas and Electricity | RC-10 Gas DSM | None | RC-10 applies only to residential and commercial buildings | None required |
| Electricity -3 Stabilized Load Growth | RC-1, RC-2: High Performance State and Local Government Buildings, Schools | None | Typically there is very little overlap between utility/EDC programmatic activity with government green building programs | None required |
| Electricity -3 Stabilized Load Growth | RC-6 Lighting | Electricity-3 | Lighting accounts for a significant portion of 2025 reductions in ACEEE et al (2009) p. 143 | All residential and commercial reductions in Electricity 3 were eliminated due to overlaps from RC-3 and RC-4. Lighting reductions kept in RC-6. |
| Electricity -3 Stabilized Load Growth | RC-7 Cool Roofs | Electricity-3 | Cool roofs accounts for 230 of 17,260 GWh commercial reductions in 2025 in ACEEE et al (2009) p. 143 | All residential and commercial reductions in Electricity 3 were eliminated due to overlaps from RC-3 and RC-4. Cool roof reductions kept in RC-7. |
| RC-8 Appliance Standards | Electricity-3 Stabilized Load Growth | Electricity-3 | Lighting accounts for a significant portion of 2025 reductions in ACEEE et al (2009) p. 143 | All residential and commercial reductions in Electricity 3 were eliminated due to overlaps from RC-3 and RC-4. Appliance reductions kept in RC-8. |
| Electricity-9 Combined Heat and Power | Industry-2 Industrial Gas and Electricity | None | Industry 2 does not target CHP specifically. In addition, the ACEEE et al (2009) report identifies between 10,000-13,000 GWh of non-CHP electricity efficiency in the industrial sector by 2025. The 2025 target under Industry 2 is only 7,900 GWh. This means that the state can fulfill the targets under Industry 2 without including overlaps for CHP from the electricity CHP workplan. | None Required |
| Forestry-9 Biomass Thermal Energy Initiatives | Electricity-9 Combined Heat and Power | None | Forestry-9 quantifies the GHG benefits and costs of utilizing biomass to power combined heat and power applications, and also includes Fuels for Schools | The CHP units deployed under Electricity 9 are assumed to operate on natural gas and thus there is no overlap with biomass as a fuel for Forestry-9 CHP. |

Appendix C: Generation Cost Assumptions and Sources

SUPPLY SIDE ASSUMPTIONS

Fuel prices: U.S. EIA, AEO 2009 (April 2009 update related to federal stimulus), Table 12 - prices for coal and natural gas for electric generation in the Middle Atlantic region.   http://www.eia.doe.gov/oiaf/aeo/supplement/stimulus/regionalarra.html.

- Nuclear fuel prices are based on NYSERDA fuel costs [placeholder].

- Biomass fuel costs assumed to be $5.78 /MMBTU.

- Waste coal prices are based on a study for U.S. EPA (see waste coal assumptions below).

- Municipal solid waste fuel prices are placeholders.

- LFG fuel costs are assumed to be $1/MMBTU for gas collection and treatment.

Equipment life:  We assume a 30-year life for all technologies except nuclear which has an estimated 50 year life..

Cost of capital: 10% weighted average cost of capital with a 50% debt and 50% equity proportion.  Cost of debt is 8% and cost of equity is 12% for all technologies.

Assumed tax credit over life of technology:  Available federal tax credits are assumed to apply to relevant generation units over the life of the plant, though the federal production tax credit applies to different renewable fuels over different periods of generation.  We assume 2007-level tax credits.  For biomass technologies, we assume the federal tax credit for open-loop biomass.  For PV, which receives a federal investment tax credit in lieu of production tax credit eligibility, and the federal government currently permits interchanging the PTC with the ITC, we assume a levelized level of tax support similar to that for wind, which was 2 cents/kWh in 2007.  DSIRE database (www.dsireusa.org) for federal tax incentives.  For small hydro, we apply the federal production tax credit for small hydropower facilities (irrigation and hydro installation at dams previously without power generation).  Federal nuclear tax credit is assumed to be $18 in 2009 and discounted at the estimated inflation rate of 2% per year.

Table 1: Summary of 2020 Costs

| Generation Modeling Assumptions | 2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Fuel Cost $/MMBTU (Waste coal in $/MWh) | Capital Cost $/kW | Capacity Factor | Tax Credits | Integration Cost | Generation Cost $/MWh |
| Coal (new supercritical) | $ 2.02 | $ 2,427 | 85% | $ - | $ - | $ 61.57 |
| Coal (existing pulverized) | $ 2.02 | $ 801 | 56% | $ - | $ - | $ 46.71 |
| Waste Coal | $ 8.92 | $ 2,460 | 85% | $ - | $ - | $ 50.10 |
| IGCC | $ 2.02 | $ 3,280 | 85% | $ - | $ - | $ 72.37 |
| IGCC with carbon capture | $ 2.02 | $ 4,662 | 85% | $ - | $ - | $ 98.12 |
| CCGT | $ 7.27 | $ 1,158 | 85% | $ - | $ - | $ 70.77 |
| Combustion NG (peaker) | $ 7.27 | $ 657 | 50% | $ - | $ - | $ 84.85 |
| Combustion NG (existing peaker) | $ 7.27 | $ 217 | 50% | $ - | $ - | $ 71.14 |
| Nuclear | $ 1.03 | $ 7,310 | 90% | $ (13.72) | $ - | $ 109.21 |
| Biomass Co-Firing | $ 5.78 | $ 461 | 85% | $ (10.00) | $ - | $ 51.44 |
| Biomass Gasification | $ 5.78 | $ 2,104 | 85% | $ (20.00) | $ - | $ 83.11 |
| PV | $ - | $ 4,218 | 13% | $ (20.00) | $ - | $ 383.24 |
| Hydro repower | $ - | $ 1,603 | 50% | $ - | $ - | $ 45.43 |
| Small hydro | $ - | $ 2,098 | 30% | $ (10.00) | $ - | $ 34.79 |
| Wind | $ - | $1,412 | 27% | $ (20.00) | $ 4.50 | $ 59.40 |
| Landfill gas | $ 1.00 | $ 1,300 | 80% | ($10.00) | $ - | $ 35.74 |
| Municipal Solid Waste | $ 2.14 | $ 5,950 | 85% | ($10.00) | $ - | $ 144.15 |
| CCS Retrofit Pulv Coal | $1.98 | $2,141 | 85% | $ - | $ - | $84.94 |
| Avoided Cost of Generation $/MWh (90% existing coal, 10% existing gas peakers) |  |  |  |  |  | $ 49.15 |

PULVERIZED COAL (EXISTING)

- Capital cost of $800 Kw means that existing coal fleet is assumed to be nearly fully depreciated (versus $2,400 kw for new coal).  Fixed costs include unallocated depreciation, boiler modifications, emissions equipment, or newer coal plants in the PA coal fleet.  O&M costs include compliance with New Source Review standards.

- Heat rate: 10,307 for all years.  This is the generation weighted average for PA’s coal fleet for 2005. Source: eGrid 2007

- O&M cost:  Both fixed and variable based on Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008)..

- Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  From Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).Heat rate:  Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

PULVERIZED COAL (NEW SUPERCRITICAL)

- Capital and O&M costs include compliance with New Source Review standards

- Capital cost:  Overnight total plant cost based on Congressional Research Service’s Power Plants: Characteristics and Costs , p. 97 (November 2008).  The CRS study includes data from numerous planned plants as well as U.S. EIA data on operations and future cost trends.

- O&M cost:  Both fixed and variable based on Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008)..

- Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  From Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).Heat rate:  Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

IGCC--Coal

- Capital cost:  Total plant cost and interest during construction data from Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

- The analysis assumes no IGCC plants until 2015.

- O&M cost:  Both fixed and variable based on Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

- Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  From Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

- Heat rate:  ICF Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

IGCC WITH CARBON CAPTURE—

- Capital cost:  Total plant cost and interest during construction data from Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).  The study draws upon work from MIT’s 2007 Future of Coal study.

- The analysis assumes no IGCC with carbon capture plants until 2020. O&M cost:  Both fixed and variable based on Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  From Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).Heat rate:  ICF Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

NATURAL GAS – COMBINED CYCLE

- Capital cost: Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).O&M cost:  Both fixed and variable based on Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).

- Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008).Heat rate:  ICF Electric Modeling Assumptions for NYSERDA, p. 82. ICF's values are for 2010, 2015, 2020 and 2025, with straight-line extrapolation applied in this analysis for interim years Congressional Research Service’s Power Plants: Characteristics and Costs, p. 97 (November 2008)..

NATURAL GAS – COMBUSTION (PEAKER)

Capital cost: Energy and Environmental Economics Inc. GHG Modeling for the California Public Utility Commission, New Natural Gas Combustion Turbine Generation, Resource, Cost, and Performance Assumptions, version 3 (October 2007), p. 3.  O&M cost:  Both fixed and variable based on Energy and Environmental Economics Inc. GHG Modeling for the California Public Utility Commission, New Natural Gas Combustion Turbine Generation, Resource, Cost, and Performance Assumptions, version 3 (October 2007), p. 3.

- Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  Assumption

- Heat rate:  Energy and Environmental Economics Inc. GHG Modeling for the California Public Utility Commission, New Natural Gas Combustion Turbine Generation, Resource, Cost, and Performance Assumptions, version 3 (October 2007), p. 3.

NATURAL GAS – COMBUSTION (EXISTING PEAKER)

- Capital cost of $217 kw means that existing gas fleet is assumed to be nearly fully depreciated (versus $650 kw for new peaking gas).  Fixed costs include unallocated depreciation, boiler modifications, emissions equipment, or newer gas plants in the PA coal fleet.  O&M costs include compliance with New Source Review standards.

- Heat rate: 8,131 is average for all NG plants in PA for 2005. Source: eGrid 2007

- Transmission cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- Capacity factor:  Same capacity factor for new peaker.

NEW NUCLEAR PLANT

- Capital cost:  Based on Center for Climate Strategies ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-32. We assume new nuclear does not come on-line until 2020 per ICF Electric Modeling Assumptions for NYSERDA, p. 82.

- Transmission cost:  Lower range of potential interconnection costs from ICF Electric Modeling Assumptions for NYSERDA, p. 83.

- O&M cost:  Both fixed and variable based on Based on ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-32.

- Capacity factor:  Based on Based on ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-32.

- Heat rate:  ICF Electric Modeling Assumptions for NYSERDA, p. 82.

- Tax credit: Federal tax credit of $18 ($2009) is applied to new advanced nuclear plants applies to the first eight years of plant operation. Because the tax credit is not adjusted for inflation by the US government, its real value is assumed to decline by 2% year starting in 2009.

NUCLEAR UPRATE

- Capital cost:  Based on FPL Energy’s proposed uprate of its Turkey Point and St. Lucie pressurized water reactor units to be completed in 2011.  http://www.fpl.com/environment/nuclear/power_uprate_faq.shtml

Such reactors exist at the Beaver Valley and Three Mile Island plants in Pennsylvania.

- Transmission cost:  Default value of $25/kW.

- O&M cost:  Assumes same variable O&M cost as new nuclear plant capacity in this analysis, but no fixed O&M due to addition to existing capacity and low overall O&M cost of uprates.

- Capacity factor:  Based on Based on ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-32.

- Heat rate:  ICF Electric Modeling Assumptions for NYSERDA, p. 82.

BIOMASS CO-FIRING

- Capital cost:  Based on Black and Veatch Economic Impact of Renewable Energy in Pennsylvania (2004), p. D-15, for 2-10% co-firing in pulverized coal plant.   Costs vary by boiler type and biomass percentage of total generation in a unit.

- Transmission:  No additional transmission investment is assumed.

- Fixed O&M Cost:  Based on Black and Veatch Economic Impact of Renewable Energy in Pennsylvania (2004), p. D-15, for 2-10% co-firing in pulverized coal plant.

- Variable O&M Cost:  Based on Based on Black and Veatch Economic Impact of Renewable Energy in Pennsylvania (2004), p. A-9, for 2-10% co-firing in pulverized coal plant.  The $0 value falls between other estimates, including negative costs (PS technology mitigation template summary for NYSERDA) and positive costs (ICF).

- Fuel cost:  Assumption of $2/mmBtu.

- Capacity factor:  Based on pulverized coal capacity factor in this analysis (85%).

- Heat rate: Assumption based on heat rate for supercritical pulverized coal plant discussed above.

BIOMASS GASIFICATION

- Capital cost:  Total plant cost and interest during construction data from ICF Electric Modeling Assumptions for NYSERDA, p. 91.  ICF assumes no biomass gasification plants until 2015, and estimates $1,920 in 2015, $1,860 in 2020 and $1,759 in 2025. 2026-2030 costs based on average annual change in cost between 2020-2025.

- O&M cost:  Both fixed and variable based on ICF Electric Modeling Assumptions for NYSERDA, p. 91.  ICF's values are for 2015, 2020 and 2025, with straight-line extrapolation applied in this analysis for interim years.

- Transmission cost:  Assumes same as for a CCGT per ICF Electric Modeling Assumptions for NYSERDA.

- Fuel cost:  Assumption of $2/mmBtu.

- Capacity factor:  Assumption of 85%.

- Heat rate:  ICF Electric Modeling Assumptions for NYSERDA, p. 91.

PV (crystalline)

- Capital cost:  ICF Electric Modeling Assumptions for NYSERDA, p. 91.  ICF estimates $4,289 in 2010, $4009 in 2015, $3,729 in 2020 and $3,391 in 2025.  Interim values are based on straight-line reduction within each 5-year period.  2026-2030 values based on average cost reduction between 2021 and 2025 (1.9%/year).

- Transmission:  Assumes distributed solar.  Central-station PV will entail more cost.

- O&M:  ICF Electric Modeling Assumptions for NYSERDA, p. 91.

- Capacity factor:  Based on PV Watts Version 1, using the ACEEE study of PV potential in PA (December 2008) for locations (Pittsburgh = 20% of all capacity, Philadelphia = 32%, rest = 48%). PV Watts estimates a 12.5% capacity factor for Pittsburgh, 13.8% capacity factor for Philadelphia, and we use Williamsport capacity factor of 12.6% for rest of state, with weighted average.

HYDRO REPOWER

- The assumptions below are based on new conventional hydropower.  However, the values fall within the range of the high variation in values for "incremental hydro" found in Black and Veatch’s Economic Impact of Renewable Energy in Pennsylvania (2004).

- Capital cost:  Based on U.S. EIA's Annual Energy Outlook 2007, Table 39 for new conventional hydropower.  2005 dollars. Capital costs were within the range of hydro upgrades considered in Avista (Washington, Montana) 2007 IRP ($1,478 to $2,168) so we retain it here, recognized the high uncertainty of such costs (as expressed by Avista in its IRP).

- Transmission cost: Default assumption of $25/kW similar to the majority of other technologies analyzed

- O&M cost:  U.S. EIA AEO 2007 for new conventional hydropower.

- Capacity factor: Assumption, p. 115, for new conventional hydropower.

SMALL HYDRO

- Capital cost:  Based on 2009 capital costs in ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-7.

- Transmission cost:  Default assumption of $25/kW similar to the majority of other technologies analyzed.

- O&M cost:  Based on ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-7.

- Capacity factor:  ESD Policy Options Document (September 23, 2008) for the Florida Governor's Action Team on Energy and Climate Change, Energy Supply and Demand Technical Work Group,  p. A-8.

WIND

- Wind capital cost:  ICF Electric Modeling Assumption for NYSERDA, p. 91. Assumes 1% reduction in costs starting in 2010 per ICF study (p. 92)

- Wind O&M cost:  ICF Electric Modeling Assumption for NYSERDA, p. 91.

- Wind transmission cost:  ICF Electric Modeling Assumption for NYSERDA, p. 95.  Assumes "Step 1" transmission which presumes easiest combination of terrain and distance, and which represent 64% (32,411 MW) of modeled resources in PJM by ICF.

- Wind capacity factor:  Based on averaging of all Class 3-5 wind resources for PJM in ICF study (p.97) for all levels of transmission difficulty.

- Integration costs:  Based on the Midwest Integration Cost Study in 2006 which found a 25% penetration of wind in Minnesota (MISO) leads to $4.5/MWh in total integration costs.  Cost is applied to all units of wind in this study, which is conservative for lower penetrations of wind compared to total generation. See http://www.awea.org/newsroom/releases/Groundbreaking_Minnesota_Wind_Integration_Study_121306.html.

COMBINED HEAT-AND-POWER ASSUMPTIONS

See CHP Workplan

WASTE COAL

- We assume that waste coal is consumed by advanced fluidized bed coal plants, which can handle low-grade fuels more effectively compared to pulverized coal.

- Capital cost:  Based on advanced fluidized bed coal-fired plant from EPRI Program on Technology Innovation: Integrated Technology Options (November 2008), p. 4-5.  ICF relies in Black and Veatch’s Economic Impact of Renewable Energy in Pennsylvania for its data.

- Transmission cost:  Default assumption of $25/kW.

- Fixed and Variable O&M:  Based on ICF’s Technical Support Document: Waste Coal-Fired Units in the CAIR and CAIR FIP, p. 8.  Originally in 1999 dollars.

- Capacity factor:  on ICF’s Technical Support Document: Waste Coal-Fired Units in the CAIR and CAIR FIP, p. 9.

- Fuel costs:  Based on Technical Support Document: Waste Coal-Fired Units in the CAIR and CAIR FIP. The study uses U.S. EIA waste coal price forecast data and assumes heat content of 8,000 Btu/pound, combined with the heat rate assumption used in this analysis (10,200 Btu/MWh)

- Heat rate:  ICF Technical Support Document: Waste Coal-Fired Units in the CAIR and CAIR FIP.

LANDFILL GAS

- Capital cost:  Based on U.S. EPA’s Landfill Methane Outreach Program’s  LFGE Project Development Handbook, p. 4-5, capital cost for internal combustion engines above 800 kW.

- Transmission cost:  Default assumption of $25/kW similar to the majority of other technologies analyzed.

- O&M cost:  Based on U.S. EPA’s Landfill Methane Outreach Program’s LFGE Project Development Handbook, p. 4-5, O&M costs for internal combustion engines above 800 kW.

- Capacity factor:  Black and Veatch’s Economic Impact of Renewable Energy in Pennsylvania, p. D-18.

MUNICIPAL SOLID WASTE

- Capital cost:  Based on Black & Veatch’s Renewable Energy Technology Assessments for Kau’I Island Utility Cooperative, p.7-8.  Assumes a combined biomass and trash facility with separate fuel streams, boilers and steam cycles for each feedstock.

- Transmission cost:  Default assumption of $25/kW similar to the majority of other technologies analyzed.

- O&M cost:  Black & Veatch’s Renewable Energy Technology Assessments for Kau’I Island Utility Cooperative, p.7-8

- Capacity factor:  Assumption of 85%.

Fuel cost:  Black & Veatch’s Renewable Energy Technology Assessments for Kau’I Island Utility Cooperative, p.7-8.  Fuel costs are highly dependent on trash tipping fees, which are not incorporated in this fuel cost assumption.

5% EFFICIENCY UPGRADES FOR EXISTING COAL-FIRED PLANTS

Cost and efficiency improvements:  Based on cost estimates on an avoided CO2 emissions basis in the Australian Greenhouse Office (January 2000) Report, Integrating Consultancy Efficiency Standards for Power Generation.  The study’s efficiency improvement estimates for several measures (excluding reduction of turbine gland leakage and low excess air operation) was corroborated in National Energy Technology Laboratory’s Reducing CO2 Emissions By Improving the Efficiency of the Existing Coal-Fired Power Plant Fleet (July 2008)

Source: ACEEE et al. (2009). Energy Efficiency, Demand Response, and Onsite Solar Energy Potential in Pennsylvania. April. P. 29. page 48.  http://www.aceee.org/pubs/e093.htm

Source: Quantec. (2008). Assessment of Energy and Capacity Savings Potential in Iowa

Prepared for The Iowa Utility Association. February 15. p. I7-I10  No web link available.

Source: ACEEE et al. (2009). Energy Efficiency, Demand Response, and Onsite Solar Energy Potential in Pennsylvania. April. P. 29. page 48.  http://www.aceee.org/pubs/e093.htm

? Source: ACEEE et al. (2009). Energy Efficiency, Demand Response, and Onsite Solar Energy Potential in Pennsylvania. April. P. 14. page 48.  http://www.aceee.org/pubs/e093.htm

http://www.puc.state.pa.us/electric/electric_alt_energy.aspx

http://www.puc.state.pa.us/electric/electric_alt_energy.aspx

US EPA.  Final Report on U.S. High Global Warming Potential (High GWP) Emissions 1990-2010: Inventories, Projections, and Opportunities for Reductions.  Chapter 3:  Cost And Emission Reduction Analysis Of Sf6 Emissions From Electric Power Transmission And Distribution Systems In The United States. http://www.epa.gov/highgwp/pdfs/chap3_elec.pdf

http://www.epa.gov/highgwp/pdfs/chap3_elec.pdf p. 3-3.

http://www.epa.gov/electricpower-sf6/documents/sf6_2007_ann_report.pdf  page 3.

http://www.epa.gov/highgwp/pdfs/chap3_elec.pdf  Exhibit 3.4.

The modeling done for the RGGI states cost approximately $1 million.  The CCAC does not have the resources to perform this type of analysis.

http://www.rggi.org/docs/program_summary_10_07.pdf. pp. 6-11.

http://apps1.eere.energy.gov/states/state_news_detail.cfm/news_id=12212/state=PA

From an email from Joe Sherrick at DEP on June 17, 2009.

US EIA. (2001). Biomass for Electricity Generation. Adjusted to $2007. http://www.eia.doe.gov/oiaf/analysispaper/biomass/pdf/biomass.pdf

http://www.flclimatechange.us/ee.cfm