Please Note: The following information has been extracted from the specification

currently used by the Texas Department of Transportation.

SPECIAL SPECIFICATION ITEM

TOTAL MAINTENANCE AND OPERATION OF HIGHWAYS

1.  Description.  This Item shall govern for the complete maintenance and operation of highways, including all the existing appurtenance and future additions.  This includes mainlane roadways, frontage roads, shoulders, ramps, intersections, roadsides, bridges, rest areas, picnic areas, weigh stations, traffic operations, etc.  The limits of this work are more fully described on the attached plans.

2.  General.  It is the intent of this specification that the Contractor shall relieve the Department of all duties traditionally performed by the Department in maintaining and operating the highways. It shall be the responsibility of the Contractor to take measures to insure that they are completely aware of the traditional functions of the Department.  It is anticipated that no change orders, except for contract extension, changes in governmental policy, changes in state or federal statutes, or catastrophic event emergency reimbursement as shown under Article 5.0 will be executed during the course of this contract.

(1) Department Standards.  Unless otherwise approved by the

Engineer, work performed and materials used under this contract

shall conform to the latest version of all Department manuals,

standards, specifications, statewide special specifications,

policies and procedures and their addenda.  These include, but are

not limited to, the following:

o   Highway Design Manual

o   Safety and Maintenance Manual

o   Maintenance Manual

o   Vegetation Management Manual

o   Traffic Control Standard Sheets Book

o   Traffic Operations Manual

o   Texas Standard Specifications for Construction of Highways,

Streets and Bridges

o   Manual of Testing Procedures

o   Texas Manual on Uniform Traffic Control Devices for Streets and

Highways (TMUTCD)

o   Sign Crew Field Book

(2) Coordination.  In performing work under this contract, the

Contractor shall insure that proper coordination exists with cities,

counties, state and local law enforcement, utilities, fire

departments, and other state and federal agencies, etc.

1-18

(3) Purchasing from People with Disabilities.  The Contractor shall

comply with the provisions of Chapter 122 of the Texas Human

Resources Code that are placed on the Department.  The use of

Community Rehabilitation Programs (CRP's) is outlined in this

Chapter and is strongly encouraged by the Department.

Specifically, Section 122.008. Procurement at Determined Prices.

states "A suitable product or service that meets applicable

specifications established by the state or its political

subdivisions and that is available within the time specified must be

procured from a CRP at the price determined by the council to be the

fair market price".

The Contractor will make a good faith effort to negotiate with CRP's

and the Texas Industries for the Blind and Handicapped (TIBH) for

subcontracts at a fair market price.  The Department reserves the

right to facilitate disputes involving subcontracts or potential

subcontracts with CRP's and TIBH.

(4) Existing Contracts.  Several Department contracts with CRP's

will be in effect at the beginning of this contract.  These existing

contracts will be assigned to the Contractor with all provisions in

force.  It shall be the Contractor's responsibility to assume

payments to the CRP's and/or TIBH.  These contracts, their scope and

duration are as listed in the General Notes and Specification Data

Sheet(s).  The Department will modify existing Routine Maintenance

Contracts (RMC's) to eliminate the work planned on the highway(s) in

this project and their work will be included in the scope of this

contract.

(5) Reporting.  The Contractor is required to have a personal

computer that is capable of connecting to the Department's

information systems and will report to the Department the following

information:

Work Accomplished - Using the Department's Construction and

Maintenance Contract System (CMCS), the Contractor shall report to

the Department work accomplished and unit costs.  This information

should be input daily, or as approved by the Engineer.  The report

shall use the Department's "function codes" to categorize the work

accomplished.  The report will show, by reference marker, the

following:

o   The date(s) of the work

o   Beginning and ending reference marker

o   The County

o   Function Code

o   The quantity of work and unit cost using the units of

measurement in the Department's Maintenance Management

Information System (MMIS)

Highway Condition Report (HCR) - By 8:10 AM each workday and as

changes occur, the Contractor shall report weather conditions and

any lane closures using the Department's HCR System.

2-18

Condition assessments - The Contractor will be required to perform

monthly condition assessments of all elements of the highway and

right of way.  These assessments shall be reported to the Department

each month.  The report shall be in an electronic format acceptable

to the Department.  In addition, the Contractor shall schedule and

perform night inspections in accordance with Article 5.5,

Performance Standards, and report findings and proposed repair

schedules to the Department, within one month of inspection.

Complaints/Service Requests - The Contractor shall report monthly, on

a format approved by the Department, information on any complaints

or service requests received from the public, cities, counties,

legislatures, etc. from the previous month.  This information will

include as a minimum, the following:

o   The date and time of the complaint

o   The location of the problem

o   The nature of the complaint

o   Who made the complaint

o   Date and action taken to address the complaint

Accidents/Incidents - The Contractor shall report no later than the

15th of each month on a format approved by the Department,

information from the previous month on any accident or incident

related to work being performed by the Contractor or within a work

zone including, but not limited to:

o   accidents involving any Contractor or subcontractor personnel,

equipment, barricades or tools;

o   traffic accidents within the limits or in the vicinity of any

work being performed by the Contractor or their subcontractors;

o   hazardous material spills;

o   any accident involving the Contractor or the traveling public

that causes damage to an appurtenance or fixture on the

right of way.

The information provided shall include as a minimum:

o   The date and time of the accident/incident

o   The location of the problem

o   The nature of the problem

o   All parties involved in the incident including name, address,

telephone number and their involvement (including witnesses)

o   Responsible party and insurance information

o   Action taken to address the incident

o   Documentation of traffic control in place at location.

In addition, the Contractor shall report to the Department

immediately on a format acceptable to the Engineer.

3-18

o   Accidents/incidents causing multiple fatalities, numerous

injuries or significant property damage resulting from fire,

explosion or the release of hazardous materials which

necessitates the evacuation of the immediate area, and the

closing of roads, streets or highways.

o   Highway accidents involving the deaths of five or more persons.

o   Any accident involving a school bus which results in fatalities

and/or disabling injuries.

o   Any incident that causes a major highway to be closed for more

than 24 hours, except for closures (maintenance, construction,

etc.) where the public has been notified in advance via

newspaper, radio or television announcements.

o   Any incident that causes major damage to highway facilities.

o   All bridge failures or closures.

o   Any chain reaction accident involving more than 10 vehicles,

regardless of the number of fatalities, injuries or length of

time the highway is closed.

Agreements - The Contractor shall provide the Department, copies of

all agreements between the Contractor and counties, cities,

municipalities, sheltered workshops, prisons, etc. that are

associated with the work on this contract.

(6) Traffic Signals and Illumination.  The Contractor will provide

maintenance and operations (including utility costs) of various

traffic signals and illumination as outlined in the General Notes

and Specification Data Sheet(s).

The Department has in place agreements that require various cities

to maintain and operate Department signals and illumination.  These

agreements require the Department to reimburse the cities for this

work.  Some of these existing agreements will be assigned to the

Contractor with all provisions in force.  It shall be the

Contractor's responsibility to assume payments to the cities for

maintenance and operation (including utility costs).  In addition,

the Contractor shall continue the use of these agreements for the

life of this contract, except where new or revised agreements

indicate otherwise.  These agreements, their scope, cost and

duration are as listed in the General Notes and Specification Data

Sheet(s).

During the life of this contract, should new signals or illumination

(except as noted below) be installed by the Department, the

Contractor will be responsible for their maintenance and operation

(including utility costs) for no additional compensation.  New

continuous lighting illumination systems will be the responsibility

of the city unless revised agreements indicate otherwise.

3.  Materials.  The Contractor will furnish all materials necessary to

complete this work.  The Contractor shall furnish the Engineer with

documentation indicating material compliance with Department

specifications unless otherwise approved by the Engineer.

4-18

4.  Equipment.  The Contractor shall be responsible for furnishing all

equipment, tools and machinery necessary for the proper prosecution

of the work.

5.  Scope of Work.  Excluding only those items of work listed in Article

5, Subarticle (1), "Items Excluded from the Contract", it is the

responsibility of the Contractor to perform all work required to

maintain and operate the highway and its appurtenances.  This

includes all maintenance and repair required to insure the highways

are kept in their designed and constructed or updated condition; and

all operational items of work to insure the highway functions as

intended.  The Contractor should be aware that this work also

includes items such as catastrophic repair, HAZMAT cleanup and

disposal, obtaining required permits, etc.

The Contractor shall pursue claims against third parties for damage

caused to the highway or its appurtenances.  The Contractor shall

also prepare the documentation in the required format to apply for

Emergency Relief Funds (ER) from the Federal Highway Administration

(FHWA) in the event of a Presidential Disaster Declaration.  The

funds acquired by the Department as a result of these claims or ER

projects shall be added to the Contractor's monthly payment no later

than one (1) month following the month the funds are received.

Funds to repair major damage caused by catastrophic events not

reimbursed by the FHWA or third parties will be added to the

Contractor's monthly payment after the work is completed.  The

damage shall be of the extent that it is above and beyond normal

routine or preventive maintenance and shall be a minimum of $50,000.

Examples include:  culverts or bridge components washing out, bridge

damage that causes a road closure, major erosion, etc.  The

Contractor shall submit a bid for the repair cost and written

approval to proceed shall be received from the Engineer prior to

work commencing.  The Department reserves the right to let an

Emergency Contract to repair the damage.

(1) Items Excluded from the Contract.  The Contractor will not be

responsible for the following items only:

o   Courtesy patrols

o   Traffic management devices (such as cameras, changeable message

signs, Automatic Vehicle Identification readers/antennae,

amplifier cabinets, detectors including acoustic, Vehicle Imaging

Vehicle Detection, microwave, etc.)

o   Agreements, such as utility permits, driveway permits, Multiple

Use Agreements, construction and maintenance agreements, and

other similar type agreements.

o   Logo signing

(2) Traffic Control Plans.  The Contractor must perform all work in

conformance with the Texas Manual on Uniform Traffic Control Devices

for Streets and Highways (TMUTCD) and the Barricade and Construction

5-18

Standards.  Unique work situations may require the preparation of

individual Traffic Control Plans (TCP's).  Unless otherwise approved

by the Engineer, individual TCP's shall be submitted to the Engineer

for approval a minimum of 72 hours prior to use.  Locations that

could pose a hazard to the traveling public shall be signed and

delineated using appropriate markings, such as barrels, chevrons,

object markers, etc.  Unless otherwise approved by the Engineer, a

person who is certified by the American Traffic Safety Services

Association (ATSSA) shall supervise the installation and removal of

traffic control devices.

(3) Lane Closures.  Except for emergencies, the Contractor will

provide a 72-hour advance notice to the Engineer of lane closures.

Dates and times when lane closures will not be allowed are as listed

in the General Notes and Specification Data Sheet(s).  The

Contractor shall keep the number of lane closures to an absolute

minimum and shall keep each closure to the shortest time duration

possible.  Should emergencies occur, such as vehicle accidents,

structural failures, etc., the Contractor shall take steps to open

the roadway as soon as possible.  Maximum distance of lane closures

shall be no more than two (2) miles and there shall be a minimum

distance of two (2) miles between lane closures.  No two (2)

consecutive exit or entrance ramps may be closed unless approved by

the Engineer.  The Engineer has the authority to deny a lane closure

in the case of a special event or other public activity that would

cause substantial delay to the traveling public.

(4) Public Notification.  The Contractor shall furnish and install

signs notifying the public that the highway is under private

maintenance and operation.  The sign locations, dimensions and

wording are as outlined in the General Notes and Specification Data

Sheet(s).  The Contractor shall provide information to the

Department concerning major lane closures or other information that

would be of interest to the traveling public.  Any information

provided to the press shall be routed through the Department's

Public Information Officer (PIO) in the District for release.  The

Contractor shall acquire and provide a toll free number so the

public can notify the Contractor of complaints or service requests.

(5) Performance Standards.  Listed below are performance standards,

which shall be utilized by the Contractor to schedule work.  The

safety of the traveling public is of the utmost importance and shall

take priority over any other work.  Damage that could endanger the

traveling public or cause further deterioration of the highway

system shall be repaired as soon as possible including working

nights, weekends and/or holidays, if necessary.  Other work should

be scheduled as soon as practical.  The term "immediately", as used

in the performance measures, means as soon as possible during normal

working hours.  The term "minimal", as used in the performance

measures, means damage that does not affect the safety of the

traveling public, does not cause further deterioration of the

6-18

facility, does not reduce the comfort of the traveling public or is

not unsightly.  If work is required for which a standard is not

listed, the Engineer will establish that standard as it becomes

necessary.

Each month the Department will determine if the Contractor is

meeting the required standards.  In addition, the Contractor shall

constantly monitor the condition of the highway.  If the Contractor

or Engineer discovers deficient sections, the Contractor shall

report to the Engineer measures that will be taken to correct the

situation.

Pavement Maintenance.

Asphalt Surfaces (Travel Lanes and/or Shoulders).

O   No ruts > 1/2".

o   No unsealed cracks > 1/4".

o   Patching, even and < 1/4" high or low.

o   Ride should be smooth with no discernible dip for hump and have

a score of 3.5 (2.5 for frontage roads) or more above on the

Mays Meter.

o   Potholes will be repaired immediately.

o   Base failures shall be repaired immediately.

o   No edge drop-offs > 2" and more than 50 continuous feet in

length.

o   No flushing allowed.

Concrete Pavement (Travel Lanes and/or Shoulders).

O   No unsealed cracks > 1/4".

o   No unsealed joints > 1/4" between travel lanes and shoulders.

o   Ride should be smooth with no discernible dip or hump.

o   Spalls or potholes will be repaired immediately unless otherwise

approved by the Engineer.

o   Pavement failures, punch-outs, joint failures, etc. shall be

repaired within 72 hours or discovery.  Temporary repairs shall

be made if the failure could cause a safety-related problem.

o   No edge drop-offs > 2" and more than 50 continuous feet in

length.

Roadside Maintenance.

Vegetation Management.

O   In urban areas as designated in the General Notes and

Specification Data Sheet(s), the height of grass and weeds shall

be kept between 7 inches and 18 inches.

o   In rural areas as designated in the General Notes and

Specification Data Sheet(s), the height of grass and weeds shall

be kept between 7 inches and 30 inches.

o   Spot mowing at intersections, ramps or other areas shall be

performed as needed to maintain visibility and sight distance.

7-18

o   Grass shall not be allowed to encroach into paved shoulders,

main lanes, sidewalks, riprap or curbs.

o   Wildflowers shall be preserved utilizing the guidelines in the

mowing specifications and vegetation management manuals.

Landscaped Areas.

O   All landscaped areas shall be maintained as near to their

originally constructed condition as possible.  Landscaped areas

will be as designated in the General Notes and Specification

Data Sheet(s).

o   Items include but are not limited to mowing, litter pickup,

irrigation system maintenance and operation, plant maintenance,

pruning, insect, disease and pest control, fertilization,

mulching, bed maintenance, watering, etc.

Litter and Debris Pickup.

O   The right of way shall be kept in a neat condition.

o   No more than 10 pieces of litter per roadside mile shall be

visible traveling at speed limit.

o   Dead animals that can be handled by one person shall be removed

immediately upon discovery.  Large animals shall be immediately

removed from the paved surfaces and disposed of within 24 hours

of discovery.

o   Tires or tire tread shall be removed from paved surfaces

immediately upon discovery.

O   All litter collected by the Contractor shall become the property

of the Contractor and shall be disposed of at approved solid

waste site(s).  Bagged litter shall be picked up and disposed of

on the same day of collection.

o   All vehicles used in transporting litter shall be equipped to

prevent the accumulated litter from being strewn along the

roadway.

o   The Contractor shall immediately remove large or hazardous

debris on paved surfaces.

Sweeping.

O   Significant buildup of dirt, ice, rock, debris, etc. on roadways

and/or bridges shall not be allowed to accumulate greater than

24 inches wide and/or 1/2 inch deep.

o   Shoulders, ramps, intersections and frontage roads, as shown in

the General Notes and Specification Data Sheet(s), shall be

vacuum swept at least once monthly.

Graffiti Removal.

O   Obscene or "gang related" graffiti shall be removed immediately

by a method approved by the Engineer.

o   Non-obscene graffiti shall be removed within two (2) weeks of

discovery.

8-18

Picnic Areas.

O   Picnic area locations will be shown in the General Notes and

Specification Data Sheet(s).

o   Picnic areas shall be kept clean and neat in appearance.

o   It shall be the Contractor's responsibility to perform daily any

needed repairs that may be noticed during the performance of

this contract.

o   "Lost and Found" items discovered at the picnic areas are to be

forwarded to the Engineer.

o   Litter should be removed from the picnic area grounds and

barrels a minimum of three times a week.  Litter shall not be

allowed to accumulate outside of the barrels.  All litter

collected by the Contractor shall become the property of the

Contractor and shall be disposed of at approved solid waste

site(s).  Bagged litter shall be picked up and disposed of on

the same day of collection.

o   All vehicles used in transporting litter shall be equipped to

prevent the accumulated litter from being strewn along the

roadway.

o   The height of grass and weeds shall be kept between two inches

and eight (8) inches.  The Contractor will be responsible for

replacement of vegetation damaged due to improper or careless

mowing and trimming operations or any other reason resulting in

damage or death of vegetation.

o   Weeds, grass and other undesirable growth shall be removed from

beds of plants and shrubs as needed.  In addition, trees and

shrubs shall be trimmed to create an aesthetically pleasing

appearance.

Rest Areas.

O   The number and location of rest areas, the number of attendants

and their duty hours shall be as designated in the General

Notes and Specification Data Sheet(s).

o   The rest area(s) shall be kept clean, sanitary and neat in

appearance at all times.  The rest area(s) shall be kept stocked

with toilet paper and hand soap at all times.  In addition, rest

rooms shall be kept deodorized at all times.

o   Any malfunction of any facility in the rest area(s) shall be

considered as an emergency.  The Contractor will be responsible

for all major maintenance work involving items such as the

electrical system, plumbing system, utility lines, hand dryers,

light fixtures, lavatory fixtures and toilets, and any other

major repairs necessary for operation of the rest area(s).

o   Routine maintenance shall be performed to keep the rest area(s)

in good condition including painting the facility, tables, arbor

units, trash barrels, etc.

o   Grass, trees and shrubs shall be watered during the early

morning hours.  Fertilizing will be done by the Contractor two

(2) times a year (early Spring and in mid-Fall) at rates

9-18

recommended by the manufacturer's specifications.  Fertilizer

shall be commercially available 13-13-13 or better or as

approved by the Engineer.

o   The height of grass and weeds shall be kept between two (2)

inches and eight (8) inches.

o   Weeds, grass and other undesirable growth shall be removed from

beds of plants and shrubs as needed.  In addition, trees and

shrubs shall be trimmed to create an aesthetically pleasing

appearance.

o   The Contractor will be responsible for replacement of vegetation

damaged due to improper or careless mowing and trimming

operations or any other reason resulting in damage or death of

vegetation.

o   When required, as designated in the General Notes and

Specification Data Sheet(s), operation of water and/or sewage

treatment plants shall be the responsibility of the Contractor.

Water and wastewater testing shall be performed as required by

state and national statutes by certified operators.

o   Utility costs at the rest area(s) shall be the responsibility of

the Contractor.

Tree and Brush Control.

O   Trees on the right of way, except in established non-mow areas,

shall be trimmed to allow mowers access.  Trees and brush shall

be trimmed to insure they do not interfere with vehicles or

sight distance, or inhibit the visibility of signs.

Drainage.

O   Cross road and side road drainage structures shall be maintained

with a maximum of 1/5 of the cross sectional area silted.  They

shall be maintained as originally constructed or subsequently

modified condition.  Any repair work performed to the structures

shall be as approved by the Engineer.

o   All culverts, pipes, channels, inlets, storm drain systems,

ditches, etc. and their appurtenances shall be kept clear and

functioning.

o   Ditch or channel erosion and siltation over 12 inches shall be

repaired by removing the siltation or filling erosion to bring

the right of way back to the original lines and grades.

Adequate sodding, seeding, fertilizer, erosion control blankets,

silt fences, rock berms, etc. shall be provided to allow the

area to revegetate.

Removal of Illegal Signing and Other Encroachments.

O   The Contractor shall insure that all illegal signs and other

encroachments are promptly removed from the right of way.  This

includes political signs, advertising signs, brick entrances,

vehicles, etc.  The Contractor shall utilize Department policy

10-18

to properly notify and collect reimbursement from owners of

encroachments.  Any funds collected by the Department for these

activities shall be added to the Contractor's monthly payment.

Mailbox Installations.

O   When requested by the public, the Contractor shall install

mailboxes supplied by the residents on posts and mounting

hardware that meet Department standards unless otherwise shown

on the plans.

o   The Contractor will work with mailbox owners to successfully

remove any non-standard mailbox supports that are installed on

the right of way.

Bridge Maintenance.

Overall Bridge.

O   Repairs for bridge damage, whether caused by collision, natural

disasters or normal deterioration shall be approved by the

Engineer in writing before the work is begun, except that

shoring or other temporary stabilization may be performed to

stabilize a structure before the final repair method is approved

by the Engineer.

o   When damage to a highway bridge structure or overpass over the

highway is discovered, the safety of the traveling public shall

be of immediate concern.  If there is any question about the

ability of the structure to be maintained in a safe manner,

detours shall be established immediately.

o   Structurally critical conditions must be addressed immediately

upon discovery.

o   When damage causes lane closures, work shall progress as soon as

possible to reopen the closed lane(s).

Railing.

O   Repair or replace damaged bridge rail, approach guardrail, end

treatments or attenuators within two days; install signs or

temporary railing as appropriate immediately upon discovery.

o   Damaged but functional traffic safety features will be repaired

or replaced within one (1) week.

Deck.

O   Spalls or damaged areas shall be repaired immediately or as

approved by the Engineer.

o   Drains must be clean and functional.

o   Joints shall be clean and appropriately sealed.

o   Loose armor joints shall be repaired immediately or as approved

by the Engineer.

11-18

Superstructures.

O   Steel beams should be maintained straight with minimal damage or

rust.

o   Steel fasteners should be in place, tight with none missing.

o   Concrete beams should have minimal unrepaired damage, spalls or

cracks.

Substructures.

O   The columns, pilings and caps should have minimal unrepaired

damage, spalls, cracks or scaling.

o   Bearing assemblies should be clean and functional.

o   Abutment and bent caps should be clean with minimal debris.

Channels.

O   Debris that interferes with stream flow shall be removed within

one (1) month after storm events.

o   Vegetation, including trees and brush, that interfere with

stream flow shall be removed from channels; however, vegetation

protecting or stabilizing channel banks should be allowed to

remain.

o   Riprap protection should be maintained to its original

configurations.

o   Undermining or riprap failures should be repaired within two (2)

months of discovery or as directed by the Engineer.

Embankments.

O   Embankments shall be maintained with minimal erosion.

o   Riprap (concrete slope protection) shall have all joints free

from vegetation and shall be sealed.

o   Slope failures shall be repaired by a method approved by the

Engineer.  Slopes shall approximately conform to the original

cross-section and shall be revegetated.

o   Retaining walls shall be maintained vertical, with drain holes

clear.  Reinforced earth walls shall be monitored for movement

or for loss of backfill and repairs as approved by the Engineer.

Traffic Operations.

O   Repair operational problems immediately (within four (4) hours

of discovery).

o   The signal timing and operational phasing shall be the

responsibility of others.

o   Inspect the highway traffic signal system a minimum of once

every 12 months.

o   Signal poles, controller pedestals and foundations are to be

kept in alignment.

o   Signal poles and controller cabinets are to be kept tight on

their foundation(s) or pedestal(s).

12-18

o   Traffic and pedestrian signal heads are to be kept aligned and

properly adjusted.

o   Provide alternate traffic control during a period of failure

within one (1) hour of discovery.

o   Provide maintenance personnel trained in the maintenance of

traffic signal equipment who will be available to respond to

emergency calls 24 hours a day, including Saturdays, Sundays and

holidays.

o   Provide state and local law enforcement agencies the location

and respective names and telephone numbers of individuals

responsible for emergency maintenance.

o   Document all checks and corrective actions in a separate logbook

for each intersection.

o   Check cabinet filter a minimum of once every six (6) months and

clean if necessary.  Cabinet filter shall be replaced every two

(2) years.

Traffic Detector Loops.

O   Repair operational problems within one (1) week of discovery.

o   Loop circuit's inductance must be > 50 and < 1,000

microhenries.

o   Insulation resistance must be > 50 meg ohms.

Signs (includes overhead signs).

O   Perform night sign inspections on six-month intervals and

replace non-reflective signs within 30 days.

o   New signs shall be approved by the Engineer.

o   New signs that are necessary shall be acquired and installed on

the latest standard sign mount within 30 days after approval.

o   Maintain all sign posts vertical with all break-away sign mounts

clear of silt or other debris that could impede break-away

features.  Posts shall not be rusted.

o   All signs shall be upgraded to the latest Department standard

within two (2) years after the execution of the contract.

o   Vegetation shall be trimmed to insure good visibility of signs.

o   Signs shall be replaced when more than five (5) percent of the

face is damaged or vandalized.

o   Longitudinal placement must be appropriate for motorists at

posted speed.

o   Lateral placement must be in accordance with Department's Sign

Crew Field Book.

o   Replace deficient warning and regulatory signs as soon as

possible upon discovery.

o   Damaged "Stop", "Yield", "Do Not Enter", "One Way" and "Wrong Way" signs

shall be replaced within two (2) hours of discovery.

Highway Lighting.

O   A night inspection shall be performed monthly of luminaires and

sign lighting and all deficiencies shall be repaired within one

week after inspection.

13-18

o   100 percent of access panels present and secured.

o   Non-functional lights will be repaired within a week of

discovery.

o   Luminaire poles knocked down shall be removed immediately upon

discovery, and shall be repaired or replaced within three (3)

weeks.

o   Broken or damaged transformer bases shall be replaced within

three (3) weeks of discovery.

Pavement Graphics.

O   A night inspection shall be performed on pavement symbols every

six months and all non-reflective markings should be repaired

within one (1) month after inspection.

Pavement Markings.

O   Pavement striping shall be inspected every six (6) months and

all non-reflective markings shall be restriped within

one (1) month after inspection.

o   A minimum of 50 percent of pavement striping shall be restriped

annually.

o   Placement should meet the Department's TMUTCD and Pavement

Marking Standard Sheets.

Raised Reflective Pavement Markers.

O   Lane and center line raised pavement markers shall be removed

and replaced every 12 months.

o   Pavement markers shall be inspected six (6) months after initial

installation and all non-reflective markings should be repaired

within one (1) month after inspection.

Traffic Buttons.

O   Traffic buttons shall be removed and replaced every 24 months.

Guardrail.

O   Damaged guardrail that will no longer function as designed shall

be repaired or replaced within one (1) week; remove debris and

install warning signs immediately upon discovery.

o   Damaged, but functional, guardrail shall be replaced or repaired

within one (1) month after discovery.

o   If, in the opinion of the Engineer, they are required for access

control, the Contractor shall install new post and cable fence

or guardrail.

Impact Attenuators.

O   Repair or replace badly damaged impact attenuators within

one (1) week, remove debris and install warning signs

immediately upon discovery.

14-18

o   Damaged, but functional, attenuators shall be replaced or

repaired within one (1) month.

Overhead Signs.

O   Repair or remove overhead sign structures that present a safety

hazard immediately upon discovery.

o   Replace overhead sign structures that must be replaced within

120 days upon discovery.

o   Install temporary ground-mounted signs, as necessary, within two

(2) days of removal of overhead sign structure.

Object Markers and Delineators.

O   No more than five (5) object markers or delineators shall be

defective per mile.  They will be considered defective if they

are not reflective, are not vertical or are missing.

Snow and Ice Control

o   Roadways and bridges shall be treated for snow and ice before,

during and after winter events to allow for movement of

traffic.

o   Anti-icing or deicing chemicals shall be approved by the

Engineer by October 1st of each year.  Salt shall not be allowed

unless approved by the Engineer.

6.  Contractor Performance.  If, in the opinion of the Engineer, the

Contractor is not meeting the performance standards as shown in

Article 5, Subarticle (5), "Performance Standards", or is not

performing work according to this contract, the Department may take

steps to have the work corrected.  This may include the use of

Emergency Contracts.  Once the Contractor is notified that the

Department is taking corrective action, the Contractor shall refrain

from taking steps of their own.  The costs associated with these

measures will be deducted from any monies due the Contractor.  In

addition, liquidated damages in the amount of $5,000 per working day

(starting with the for mentioned notification), during the work

correction period, will be deducted from the amount due the

Contractor.

7.  Termination.  This is an experimental project.  If both parties to

the contract agree in writing to terminate the contract, the

Department shall prepare a termination agreement, and the contract

shall end 30 days after the date of the last signature.

8.  Measurement.  This Item will be measured by the "Lump Sum", as the

work progresses.

9.  Payment.  The work performed and materials furnished in accordance

with this Item and measured as provided under "Measurement" will be

paid for in partial payments in accordance with the following

schedule, utilizing the unit price bid for "Total Maintenance and

15-18

Operation of Highways".  This price shall be full compensation for

this work and for furnishing all labor, equipment, materials, fuel,

tools and incidentals necessary to complete the work for a five-year

period.

Should construction or reconstruction project(s) occur involving

portions of highways covered by this contract, the Contractor shall

be relieved of duties along those portions of highways for the

duration of the construction project.  Monthly payments to the

Contractor will be reduced by an amount equal to the normal monthly

payment divided by the total lane miles in the contract multiplied

by the total lane miles in the construction project(s).

Payment Schedule.  Monthly payments shall be made by multiplying the

"Lump Sum" bid by the payment schedule percentage and deducting any

amounts as indicated above under "Contractor Performance".

Payment           Cumulative           Cumulative

Schedule           Payment                Time

Month          Percent            Percent              Percent

1             1.850%              1.850%               1.667%

2             1.850%              3.700%               3.333%

3             1.850%              5.550%               5.000%

4             1.850%              7.400%               6.667%

5             1.850%              9.250%               8.333%

6             1.850%             11.100%              10.000%

7             1.850%             12.950%              11.667%

8             1.850%             14.800%              13.333%

9             1.850%             16.650%              15.000%

10             1.850%             18.500%              16.667%

11             1.850%             20.350%              18.333%

12             1.850%             22.200%              20.000%

13             1.500%             23.700%              21.667%

14             1.500%             25.200%              23.333%

15             1.500%             26.700%              25.000%

16             1.500%             28.200%              26.667%

17             1.500%             29.700%              28.333%

18             1.500%             31.200%              30.000%

19             1.500%             32.700%              31.667%

20             1.500%             34.200%              33.333%

21             1.500%             35.700%              35.000%

22             1.500%             37.200%              36.667%

23             1.500%             38.700%              38.333%

24             1.500%             40.200%              40.000%

25             1.530%             41.730%              41.667%

26             1.530%             43.260%              43.333%

27             1.530%             44.790%              45.000%

28             1.530%             46.320%              46.667%

29             1.530%             47.850%              48.333%

30             1.530%             49.380%              50.000%

31             1.530%             50.910%              51.667%

16-18

Payment           Cumulative           Cumulative

Schedule           Payment                Time

Month          Percent            Percent              Percent

32             1.530%             52.440%              53.333%

33             1.530%             53.970%              55.000%

34             1.530%             55.500%              56.667%

35             1.530%             57.030%              58.333%

36             1.530%             58.560%              60.000%

37             1.561%             60.121%              61.667%

38             1.561%             61.682%              63.333%

39             1.561%             63.243%              65.000%

40             1.561%             64.804%              66.667%

41             1.561%             66.365%              68.333%

42             1.561%             67.926%              70.000%

43             1.561%             69.487%              71.667%

44             1.561%             71.048%              73.333%

45             1.561%             72.609%              75.000%

46             1.561%             74.170%              76.667%

47             1.561%             75.731%              78.333%

48             1.561%             77.292%              80.000%

49             1.893%             79.185%              81.667%

50             1.893%             81.078%              83.333%

51             1.893%             82.971%              85.000%

52             1.893%             84.864%              86.667%

53             1.893%             86.757%              88.333%

54             1.893%             88.650%              90.000%

55             1.893%             90.543%              91.667%

56             1.893%             92.436%              93.333%

57             1.893%             94.329%              95.000%

58             1.893%             96.222%              96.667%

59             1.893%             98.115%              98.333%

60             1.885%            100.000%             100.000%

10.  Contract Extension.  If agreed upon in writing by both parties to

the contract, the contract may be extended an additional 36 months

in accordance with the following payment schedule.  Either party to

this contract may request a revised pay schedule for the contract

extension, and if executed by change order, will replace the

following:

Payment Schedule.  Monthly payments shall be made by multiplying the

original "Lump Sum" bid by the payment schedule percentage and

deducting any amounts as indicated above under "Contractor

Performance".

Payment           Cumulative           Cumulative

Schedule           Payment                Time

Month          Percent            Percent              Percent

61             1.750%            101.750%             101.667%

17-18

Payment           Cumulative           Cumulative

Schedule           Payment                Time

Month          Percent            Percent              Percent

62             1.750%            103.500%             103.333%

63             1.750%            105.250%             105.000%

64             1.750%            107.000%             106.667%

65             1.750%            108.750%             108.333%

66             1.750%            110.500%             110.000%

67             1.750%            112.250%             111.667%

68             1.750%            114.000%             113.333%

69             1.750%            115.750%             115.000%

70             1.750%            117.500%             116.667%

71             1.750%            119.250%             118.333%

72             1.750%            121.000%             120.000%

73             1.785%            122.785%             121.667%

74             1.785%            124.570%             123.333%

75             1.785%            126.355%             125.000%

76             1.785%            128.140%             126.667%

77             1.785%            129.925%             128.333%

78             1.785%            131.710%             130.000%

79             1.785%            133.495%             131.667%

80             1.785%            135.280%             133.333%

81             1.785%            137.065%             135.000%

82             1.785%            138.850%             136.667%

83             1.785%            140.635%             138.333%

84             1.785%            142.420%             140.000%

85             1.821%            144.241%             141.667%

86             1.821%            146.062%             143.333%

87             1.821%            147.883%             145.000%

88             1.821%            149.704%             146.667%

89             1.821%            151.525%             148.333%

90             1.821%            153.346%             150.000%

91             1.821%            155.167%             151.667%

92             1.821%            156.988%             153.333%

93             1.821%            158.809%             155.000%

94             1.821%            160.630%             156.667%

95             1.821%            162.451%             158.333%

96             1.821%            164.272%             160.000%