Bending Glass and Making Glass

WEAR SAFETY GLASSES

Making a glass design

1. Use a triangular file to cut a piece of tubing about 6 inches long.

2. Fire polish each end. Place each end in the flame and rotate it until it looks smooth. Be sure to allow the first fired polished end to cool before attempting to fire polish the other end.

3. Grasp both ends of tubing. Hold the tubing in the hottest part of the burner flame. Rotate the tubing to evenly heat all sides.

4. When the heated portion gets soft and wobbly, remove glass from heat. Smoothly bend the ends of the glass tubing.

5. Continue bending the glass until you have the shape you choose (a letter, a heart, etc.)

Making an Optical Fiber

1. Cut a piece of glass rod about 6 in long.

2. Move the rod back and forth in the flame to soften at least 4 cm of the glass.

3. Use both hands to heat the center portion of the rod. Rotate while heating.
4. When rod is fairly soft and wobbly, quickly remove the rod from the burner.
5. Rotate rod so that it is vertical. Quickly spread your arms while firmly grasping the rod.

6. Check your fiber to see if it will transmit light. By shining a green laser into the rod.

Blowing a Glass Bubble

1. Cut a piece of tubing about 6 in long.

2. Fire polish one end of the tubing and allow to cool.

3. Heat the end that has not been fire polished until it is soft. Then use a pair of pliers to seal the end or push it against a ceramic tile. It is also possible to allow the end to close on its own with enough heating.

4. Heat the tubing about one cm above the sealed end until it is soft. Quickly remove the tubing from the heat and blow into the cool end while rotating the tube. Some people prefer to leave the glass in the flame while blowing. DO NOT PUT YOUR MOUTH TO THE GLASS

5. Repeat step 4 until you have a bubble with a diameter about twice that of the tubing. As the glass walls of the bubble get thinner use less air pressure to avoid bursting the bubble.

Glass Bead on a Wire

1. Cut a piece of copper wire about 3 in long.

2. Use pliers to form a small loop on the end of the wire.

3. Use pliers to hold the wire while heating. Using a Bunsen burner heat the loop end of the wire until it gets red hot.

4. Dip the hot end of the wire into borax.

5. Carefully heat the borax on the wire until it is melted and the loop fills in. Do not add more borax until the bead becomes clear and almost all bubbles are gone. The bead should have a color.

6. You may need to repeat steps 5 and 6 to fill in the loop and get a bead.