AP Physics  - Electric Circuits, DC

# Series Circuits

Rules For Resistance in Series:

- current in every part of circuit is same.

- total resistance in circuit =  sum of all resistances.

- total voltage =  sum of all voltage drops.

R=R+R2+⋅+Rn

I = I1 = I2 =    = In

V=V+V2+⋅+Vn

- find the total resistance and the current.

Find R:   R=R+R2+R3

R = 25  + 35  + 32,    	 R = 92 

Find I:   	V = IR,          I=VR		I=18V92Ω

I = 0.196 A

I = 0.20 A

- Yet another problem.  Figure out the total voltage and the current in this circuit.

find current through R1 :

V=IR	I=VR		I=12.0V45.0Ω

I = 0.267 A			This is total current

R=R+R2		R = 45.0  + 75.0 

R = 120.0 

V = IR		V = 0.267 A (120.0 A)

V = 32.0 V

Parallel Circuits   current has more than one path

Rules for Parallel Circuits:

- voltage same across each leg of parallel circuit.

- total current is =  sum of currents through each leg.

- total resistance is less than the resistance of any one branch.  Rciprocal of the total resistance is equal to the sum of the reciprocals of the resistance of each branch.

- Ohm's law applies separately to each branch.

total resistance:	1R=1R1+1R2+⋅+1Rn

R=R1R2R1+R2

I = I1 + I2 +    + In

V=V=V2=⋅=Vn

- Look at this circuit: find the total resistance, total current, current through each leg.

Basic Equations:	R=R1R2R1+R2		V = IR

I = I1 + I2		V = V1 = V2

Resistance.

R=R1R2R1+R2				R  = 24.79 

R  = 25 

Current:

V = I R, 		  I=VR		I=45V25Ω	 = 1.8 A

Current through each branch:

V = I R,   	I=VR		I1=45V85Ω	I1 = 0.53 A

I2=45V35Ω		I2 = 1.3 A

Equivalent Circuits:

Another: