Calculate the Heat Flow Rate of the Wall and Window?

Equation Needed:  A = Length x Height;  Resistance = thickness / Thermal Conductivity x Area

Procedure:

- First find the area of the outside wall by measuring its height and length and multiplying the two.  (Measure in centimeters)  Record your answer.  Area of outside wall = _________________

- Than measure the thickness of the block, the insulation between the block and brick, and the thickness of the brick.  Record your answers:

  - Thickness of block = ____________ centimeters

  - Thickness of brick = ____________ centimeters

  - Thickness of Insulation = ___________ centimeters

- Look on page 160 and 221 of your textbook and record the Thermal conductivities of each of the materials:

  - Thermal conductivity of a block (cement) Page 160 = ___________ cal cm / s cm3 C

  - Thermal conductivity of the brick (page 221 on bottom of page) = ___________ cal cm / s cm3 C

  - Thermal conductivity of the insulation (page 221) = ___________ cal cm / s cm3 C

- Calculate the resistance of each of the materials using the equation:

# Resistance = thickness / Thermal Conductivity x Area

  - Resistance of the block = __________________________________

  - Resistance of the brick = __________________________________

  - Resistance of the Insulation = ______________________________

- Add up the resistance of the block, brick, and insulation and record the total resistance.  Total Resistance = ____________________

- Measure the temperature in Celsius inside and record = ______________

- Measure the temperature in Celsius outside and record = _____________

- Finally, the heat flow rate through the wall is equal to the change in temperature divided by the total resistance.

- The rate of heat flow from the outside to the inside through my wall = ________________

## Procedure 2

- Find the area of one window in centimeters = _______________

- Measure the thickness of one window in centimeters = ___________

- Look on page 160 of textbook to find the thermal conductivity of glass = ___________

- Find the resistance of glass using equation:  resistance = thickness / thermal conductivity x area

- Use the previous temperature measurements and use the equation: heat flow rate = change in temperature / resistance to find the heat flow rate from a window = _________________

## Conclusion

- Which has a greater heat flow rate, the wall or window?  Which is a better insulator?  How could we improve the insulation of both the window and the wall?

- Which of the following is causing the transfer of heat; conduction, convection, or radiation?  Explain why each of these are either causing a transfer of heat or why they are not causing a transfer of hear?