Studio in Multi Media

Pinhole Camera

For this project, you are to create a pinhole camera out of a shoebox with your partner and make two unique completed photographs of different subjects/ backgrounds.  Think of the various elements and principles of art for a dynamic composition!

Bear in mind the following information:

- The initial photograph that you are taking with the pinhole camera is a paper negative.  You will then take that and make a contact print, just as we did with your photograms, for the final print.

- There will be some trial and error involved with the making of the paper negative, as it’s an imperfect science.  The time will vary from pinhole camera to pinhole camera, depending on the light condition and how it’s constructed, so there’s no specific time down to the second of exposure time, just general time frames.  You may have to experiment a few times to get the right exposure, which is why you must keep a record of your times.

- The brighter it is outside, the less time is needed.

- The darker it is, the more time you need.

C. The size of the pinhole (the Aperture) also determines how the

paper negative comes out.

1. A bigger hole lets more light in & makes the exposure

quicker, but the image will not be very sharp.

2. A smaller hole lets less light in & it will take longer to

make an exposure, but the image will be sharper.

- If your original paper negative is white or has only a faint image:

- The paper may have loaded backwards in the camera.

- The shutter was not open during exposure.

- The exposure time was too short.

- If your original paper negative is too dark or black:

- The camera may have a light leak.  Find the additional light sources on the camera and cover with duct tape.

- The paper may have been exposed to white light.

- The exposure time was too long.

You will be graded based on the following:

Time on task- 10 points			Construction of Shoebox- 20 points

Developing Quality of Prints- 20 pts	Print Quality- 20 points

Creativity of Photograph- 20 points	Matting- 10 points

Name:					         Pinhole Camera Time Records

Subject				Exposure Conditions (Sunny/cloudy?)          Exposure Time

______________________	_______________________________	____________

______________________	_______________________________	____________

______________________	_______________________________	____________

______________________	_______________________________	____________

______________________	_______________________________	____________

______________________	_______________________________	____________

______________________	_______________________________	____________

______________________	_______________________________	____________

Suggested Exposure Times:

Shade:		1:40- 3:00 minutes or longer			Sun: 30 seconds- 1 minute

Direct Sun:	8- 15 seconds

Paper Negative Exposure Times (KEEP YOUR PAPER NEGATIVE SAFE!)

Subject					   Exposure Aperture (f4?)                 Exposure Time

_____________________________     ______________________	 	____________

_____________________________     ______________________	 	____________

_____________________________     ______________________	 	____________

_____________________________     ______________________	 	____________

_____________________________     ______________________	 	____________

_____________________________     ______________________	 	____________