Resistance Power

Materials:

Alligator Clamps

Volt Meter

Resistors

Battery

Procedure:

- Connect 1 of the resistors in series with the battery.

- Measure and Record the resistance of the resistor in the data table.  ( symbol on multimeter)

- Measure the voltage before the resistor and record.  (V symbol on multimeter)

- Measure the voltage after the resistor and record.

- Remove the resistor and place the 2nd resistor in series.  Repeat the procedures and record.

- Repeat with the 3rd resistor.

Data:

Trial		Resistance		Volt before		Volt After		Power  (V / R)

1		_______		_________		_________		_______________

2		_______		_________		_________		_______________

3		_______		_________		_________		_______________

Conclusion:

- Which resistor had the highest power;  the largest resistor or the smallest resistor?

- How did the size or value of the resistor affect the Voltage?

- What factors affect Power?

- If your resistance is increased, what would happen to the power?

- If your Voltage is increased, what would happen to the power?