Tooth Tunes

Background Information:

A miniature electric motor and a speaker have the same basic components:  a coil and a permanent magnet.  When electrical impulses pas through the voice coil of a speaker, it interacts with the magnet and moves back and forth.  When a current passes through the coil of a motor, it interacts with the magnet and spins.  When sound is passed to a motor, it causes a slight vibration of the arm of the motor.  These vibrations produce sounds.

Materials:

- 1.5 Volt motor

- Earphones

- Straws

- Scissors

- Music source (cellphone works well)

Procedure:

  - Cut the earphones from its cord.  We only need the plug and the wire leading up to the earphones.

  - Separate the 2 wires that make up the cord for a distance of a few centimeters.

  - Remove about 2 cm of the insulation from the end of each wire.

  - Tightly connect the wires to an electric motor.

  - Place a straw over the tip of the motor (for sanitary purposes)

  - Plug the cord into a music source such as your cellphone and turn the volume on high.

  - You should feel the motor vibrating.  If not, check your connections.  Also, try switching your connections.

  - Clamp your teeth loosely over the the straw.

  - Use your fingers to close your ears.

  - You should hear the music clearly.  The motor is conducting the sound to your teeth, thru the bones of your skull, and to your inner ear.

Conclusion:

- Describe what you experienced?

- Why do you hear the music when you clamp with your teeth?

- What happens if you turn up the music or down the music?  Why do you think this is?

- Explain how and where the sound is traveling.